Blaze EZ

User Manual

(Model: T70EZ)

ENGLISH MANUAL
 For Software Version 2.0

(July, 2016)

When there are any manual updates, 
the readme files or appendices 
will be added to the manual CD 
and website.

HIMS Inc.

4616 W. Howard Lane, Ste 960 Austin, TX 78728

Phone: 888-520-4467

Fax: 512-837-2011

Email: support@hims-inc.com

Congratulations on the purchase of your Blaze EZ! The Blaze EZ is capable of scanning printed material and reading DAISY content  as well as many standard functions of portable mainstream media devices, – but with a simplified interface utilizing just a few, intuitive, tactile buttons rather than using a touch screen!

Before using the Blaze EZ, you should read the entire manual to familiarize yourself with the functions of the Blaze EZ. The Blaze EZ contains many programs with a variety of functions, thus, reading the entire manual will allow you to operate the Blaze EZ to its fullest potential.

If you are unable to find a solution to a problem within the manual, or if you need assistance with the Blaze EZ, please email us at supprot@hims-inc.com. You may also visit us on the web at www.himsintl.com; or, you can call us at +82-42-864-4460.

Copyright 2016. All RIGHTS reserved, HIMS.

This User Manual is protected by copyright belonging to HIMS, with all rights reserved. The User Manual may not be copied in whole or part, without consent from HIMS.

Safety Precautions

For your safety and protection of the Blaze EZ, please read and abide by the following safety precautions.

  - The input voltage of the AC adapter is 100V - 240V, and the output is DC 5V / 2A.

  - When the Blaze EZ is shipped, the battery may be packaged separate from the unit to avoid damage. When you use the Blaze EZ for the first time, you may need to open the package, remove the battery, and insert it into the battery slot at the rear panel of the unit. (See section 1, introduction, for more details on inserting the battery.) It is possible that the dealer or distributor from whom you purchased the Blaze EZ has already done this for you. The battery may not be fully charged when it is shipped. Before you use the Blaze EZ, you should ensure the battery is fully charged.

  - When you use the Blaze EZ with the battery for the first time, the battery status may not be displayed accurately. To avoid this, leave the Blaze EZ connected to the AC adapter for about three hours with the unit turned on. If you want to use the Blaze EZ immediately, you may use it while it is charging.

  - If you want or need to remove the battery from the unit, power the unit off, and then remove the battery. If the unit is connected to AC power, before re-inserting the battery in to the unit, make certain that the power is off.

There is a risk of explosion if battery is replaced by an incorrect type. Please make certain to use only battery packs produced specifically for the Blaze EZ by HIMS. Please dispose of used batteries according to the instructions.

  - When you are using the Blaze EZ on battery power, the status of the remaining battery power is announced when the battery charge is low. When the battery’s charge falls below 10 percent, connect the AC adapter to the unit for recharging. If the remaining battery charge drops to five percent, and the unit is not connected to the AC adapter, the Blaze EZ shuts down one minute after the announcement of the battery’s status. The amount of battery life per charge varies depending on the options you have set and the number and type of tasks you are running.

  - Your Blaze EZ needs to be handled with care. The Blaze EZ is a very delicate machine. Please handle the unit in a proper, careful manner. The Blaze EZ is very sensitive to dust. Make sure to keep the Blaze EZ away from dusty environments.

  - Do not use the Blaze EZ in a moisture-rich environment such as a bathroom or shower. This is to prevent damage caused by moisture getting into the device. Do not touch the AC adapter with wet hands.

  - Do not leave your Blaze EZ in closed or high temperature environments such as inside a car on a hot summer day, as the battery attached to the Blaze EZ may be damaged or catch fire. Please do not let your Blaze EZ remain in such environments for long periods of time.

  - To prevent possible damage to your hearing, do not listen to audio at high volume levels for long periods.

  - Do not take apart the Blaze EZ yourself. Do not have anyone else who is not authorized by HIMS take apart the Blaze EZ. If an unqualified person disassembles the unit, serious damage may occur to the Blaze EZ. If an unauthorized person disassembles the Blaze EZ, the unit is excluded from any free maintenance, and the warranty becomes void. If any liquid or external force damages the unit, it may also be excluded from free maintenance, even if the damage occurs during the warranty period.

  - Thank you for using the Blaze EZ. We value any comments or suggestions you have for our product. If you have any complaints or suggestions, please provide us with your comments on our website. We will improve our product based on your comments and suggestions.

  - This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

Caution Statement for Modifications

CAUTION: Any Changes or modifications not expressly approved by the manufacturer could void the user's authority to operate the equipment.

Mobile Device RF Exposure Statement

RF Exposure - This device is only authorized for use in a mobile application. At least 20 cm of separation distance between the transmitting antenna device and the user's body must be maintained at all times.

Model Name: T70EZ

Manufacturer Name: SELVAS Healthcare, Inc.

S/N:

This device complies with part 15 of the FCC Rules.

Operation is subject to the following two conditions:

- This device may not cause harmful interference, and

- This device must accept any interference received, including interference that may cause undesired operation.

Table oF contents

1. Introduction.	1

1.1 Main Features.	1

1.2 What's in the Box?	2

1.3 Hardware Specifications.	3

1.4 Physical Orientation to the Blaze EZ.	4

1.4.1 The Top Face.	4

1.4.2 The Left Edge.	5

1.4.3 The Right Edge.	6

1.4.4 The Top Edge.	6

1.4.5 The Bottom Edge.	6

1.4.6 The Back Face.	7

2. Getting Started.	8

2.1 Inserting and Removing the Battery.	8

2.2 Charging.	8

2.3 Powering On/Off.	9

2.4 Inserting and Removing the SD CARD.	10

2.5 Connecting to a computer.	10

2.5.1 Connecting to a PC.	10

2.5.2 Connecting to a Mac.	11

2.6 Folders used in the Blaze EZ.	12

3. Basic Functions.	13

3.1 The Home Menu.	13

3.2 Adjusting Volume and other properties.	14

3.3 Adjusting Voice Settings.	15

3.4 Checking Date  and Time.	15

3.5 Setting the Date and Time.	16

3.5.1 Manually setting the Date and Time.	16

3.5.2 Synchronizing with a Time Server.	17

3.6 Checking Power Status.	17

3.7 Setting the Sleep Timer.	17

3.8 Checking System Information.	18

4. Using the Media Player.	19

4.1 Supported Media Formats.	19

4.2 Playing Audio Files.	19

4.3 Play and Pause.	19

4.4 Moving by Track.	20

4.5 Moving by Time.	20

4.6 Changing Navigation units.	20

4.7 The menu.	21

4.7.1 Move to a specified time.	21

4.7.2 Moving by percentage.	21

4.7.3 Moving to a specified track.	22

4.7.4 Setting Media preferences	22

4.7.5 Getting Playback Information.	23

4.8 Marks.	23

4.8.1 Inserting a Bookmark.	23

4.8.2 Inserting a Voice Mark.	24

4.8.3 Inserting a Highlight Mark.	25

4.8.4 Moving to a Mark.	25

4.8.5 Deleting a Mark.	26

4.9 Adjusting Volume, speed, and Equalizer.	27

4.10 Using The Explorer.	27

5. FM Radio.	29

5.1 Playing the Radio.	29

5.2 Adjusting Radio Volume.	29

5.3 Navigating Frequencies.	30

5.4 Selecting and Moving to Channels.	30

5.5 Using the Internal Speaker.	30

5.6 Recording a Radio Broadcast.	31

6. Book Reader.	32

6.1 Supported File Types.	32

6.2 Play and pause.	32

6.3 Navigation.	33

6.4 Quick Movement.	34

6.5 Marks.	35

6.5.1 Inserting a Mark.	35

6.5.2 Inserting a Voice Mark.	36

6.5.3 Inserting a Highlight Mark.	36

6.5.4 Moving to a Mark.	37

6.5.5 Deleting a Mark.	38

6.6 Adjusting Volume and Speed	38

6.7 Switching Daisy Modes.	39

6.8 Deleting Content.	39

6.9 The Menu.	39

6.9.1 Moving to a specified time.	40

6.9.2 Moving by percentage.	40

6.9.3 Moving to a specified heading.	40

6.9.4 Moving to a specified page.	40

6.9.5 Reviewing Book or File Information.	41

6.10 Using The Explorer.	41

6.10.1 Entering and Exiting the  Explorer.	41

6.10.2 Structure of the Explorer	42

6.10.3 Searching for and Playing Content.	42

6.10.4 Continuous Playing of a Folder.	44

6.10.5 Checking the path.	44

6.10.6 Delete file or folder.	44

7. Recording.	45

7.1 Making a Recording.	45

7.2 Listening to a Recorded File.	45

7.3 Searching for and Deleting Recorded Files.	46

7.4 Setting Recording Options.	46

8. OCR.	48

8.1 Opening and Closing OCR.	48

8.2 Taking a Picture.	48

8.3view Guidance.	49

8.4 Taking continuous pictures.	50

8.5 Reading the OCR Result.	51

8.6 Importing Image Files.	52

8.6.1 Opening a Captured File.	52

8.6.2 Opening an External Image File.	52

8.7 Setting Other OCR Options.	53

8.7.1 Turning Flash On/Off.	53

8.7.2 Selecting Image Recognition File handling.	53

8.7.3 Deleting All Recognized Text Files.	53

9. Web Radio.	55

9.1 Executing and exiting Web Radio.	55

9.2 Moving by channel.	55

9.3 Searching for Web Radio Channels.	56

10. Podcasts.	58

10.1 Subscribing to Podcasts.	58

10.2 Executing and Exiting Podcasts.	59

10.3 Navigating Feeds and Podcasts.	59

10.4 Deleting Content.	60

11. Library Services.	61

11.1 Logging In.	61

11.2 Navigating content.	62

11.3 Downloading and Playing content.	62

11.4 Returning content to the Online DAISY Bookshelf.	63

12. Utilities.	64

12.1Color Reader.	64

12.1.1 opening the color reader.	64

12.1.2 Using the color reader.	64

12.2 The Alarm.	65

12.2.1 Setting an alarm.	65

12.2.2 Deleting an alarm.	65

12.3 Format.	66

12.4 Playing an Audio CD.	66

12.5 Upgrading Blaze EZ firmware.	67

12.5.1 Online Upgrade.	67

12.5.2 Offline Upgrade.	67

12.6 Initialize Options.	68

13. The options menu.	69

13.1 Global Settings.	69

13.2 Sound Settings.	70

13.3 Using a Bluetooth Audio Device.	71

13.3.1 Turning Bluetooth On/Off.	71

13.3.2Connecting a Bluetooth headset or Speaker.	71

13.3.3Disconnecting a Bluetooth Headset or Speaker.	72

13.4 Using Wireless LAN.	73

13.4.1 Checking Wireless status.	73

13.6.2 Turning Wireless On/Off.	73

13.4.3 Connecting to a Wireless network.	74

13.4.4 Deleting a Wireless Profile.	74

14. Appendices:	76

14.1. Appendix 1: The premium Accessory Pack.	76

14.1.1 Assembling the OCR stand.	76

14.1.2 The OCR remote.	78

14.1.3 The Cradle	78

14.2. Appendix 2: Using Blaze EZ with Windows Media Player and Windows XP.	79

1. Introduction.

Thank you for purchasing the Blaze EZ OCR multi-player.

The Blaze EZ is a digital audio player and portable OCR device manufactured by HIMS. It is a small, lightweight portable device which can play digital books, music files, document files, DAISY content, FM radio, and can scan and recognize printed text and text image files with the built-in 5 megapixel camera and OCR software. With the built-in Wi-fi users can also utilize Online DAISY, Web Radio and subscribe to and download podcasts.

Though the Blaze EZ is a powerful device with advanced features, it remains easy to use, even for the non-technical person, with many functions available with the press of a single button.

1.1 Main Features.

This section introduces the key features of the Blaze EZ and briefly outlines the main functions.

- Use the Media player to play a variety of multimedia file formats including music, audio books, podcasts, and your own recordings. The Media Player features navigation by several movement units including tracks and time elements, adjustment of playback speed and preamp, Equalizer settings, and more. In addition, by connecting an external CD-Rom drive with an Audio CD inserted, you can use Blaze EZ to play commercial CD's.

- Use the Book Reader to easily read Daisy books and various document file types.  Navigate by pages, paragraphs, sentences, lines, and other text units. Adjust reading speed, volume, and pitch, reading of DAISY audio and more.  In addition, by connecting an external CD-Rom, you can also use Blaze EZ to read DAISY books produced on CD.

- Listen to the FM radio, auto scan  for channels, store presets, and record for later listening.

- Use Blaze EZ's built-in camera to capture images of printed textbooks, brochures, or restaurant menus  and recognize them using the OCR program. You can also import images from other sources and recognize them as well.

- Record via an internal / external microphone, using an Apple-compatible headset microphone or line-in recording source. Record using mp3 or wave file formats and adjust a variety of options, such as microphone sensitivity and recording quality.

- Listen to media or document playback via a Bluetooth headset connection for wireless audio listening.

- Use the Built-in Wi-Fi and included Podcast application to subscribe to and download podcasts for listening on the go.

- Use the Web radio application to stream music, news and other audio content from around the world.

- Clock function: Use the Blaze EZ as your talking clock even when the power is off, just press the power button to check the current date and time.

1.2 What's in the Box?

Your Blaze EZ packing box should contain the following items:

- Blaze EZ

- Rechargeable Battery Pack

- Carrying pouch

- AC adapter

- USB Cable for connecting to PC

- USB Gender adapter

- Earbuds

- Documentation CD

- Getting Started Guide

- Optional Accessory Pack:

- OCR stand

- OCR Remote

- Cradle

- Spare Battery Pack

- Spare AC adapter

1.3 Hardware Specifications.

- User memory (internal memory): 12GB

- Micro USB OTG port

- SD card slot

- FM radio

- Bluetooth 3.0 (Bluetooth stereo headset support)

- Wi-fi (802.11B/G/N, 2.4GHz)

- 5-megapixel camera

- Headphone jack (3.5 mm, apple compatible headsets can be connected.

- Stereo microphone jack (3.5 mm, supports line-in input)

- Built-in stereo speakers

- Built-in mono microphone

- Replaceable battery (Lithium Polymer, 13 hours of continuous music playback when fully charged.

1.4 Physical Orientation to the Blaze EZ.

The Blaze EZ is a small, rectangular unit, similar in size and shape to a smart phone. It weighs just 118g, is 59.3mm wide, 116.86mm long and 16.44mm thick.

Place the Blaze in your hand or on a desk or table top in front of you so that the keypad is facing up and the long rectangular slot on the side of the unit is facing left.

Power On/Off Button

Record Button

Radio Button

Music Button

Book Button

Navigation Pad

Cancel Button

OCR Button

Explorer Button

<The Top Face>

1.4.1 The Top Face.

The top face of the unit contains the main keypad. At the top center is a slightly recessed square depression. This is the Power On-Off button. To the left and right of the Power button are the stereo speakers.

Below the Power button, toward the left is a round button. This is the Record button.

Below the Record button is a row of 3 square buttons with Braille labels. From left to right these buttons are the Media button, the Radio button, and the Books button. These buttons open the Media Player, the FM Radio, and the Book Reader respectively.

Below the program buttons is a circular navigation pad containing 5 keys: the Left, Right, Up and Down arrows, and a small round button in the center which is the Select button.

Below the navigation pad is a row of 3 buttons: 2 square buttons with a dot-like button between them. The square button on the left is the Cancel button. The Square button on the right is the OCR button. The dot-like button between them is the Explorer button. The Explorer button brings up a list of files associated with the currently open program. For example, if you have the BOOK reader opened, pressing the explorer button will bring up a list of DAISY books and documents readable by the Book Reader. The Cancel button is used to exit a program, menu or dialog.

On the very bottom of the front face are 2 LED's showing power and charging status on the left, and the internal microphone on the right.

Volume Dial

Reset Button

Key lock Switch

Voice control Button

SD card slot

<The Left and Right EDGE>

1.4.2 The Left Edge.

The left edge of the unit contains a long, thin, rectangular slot. This is the SD card slot.

1.4.3 The Right Edge.

The right side of the unit contains 3 controls. Nearest the top of the unit is the volume dial. Turn the dial away from you to increase the volume, and turn it toward you to decrease it. As you adjust the volume, you will feel the dial click past each setting, and the unit announces the volume level as a number.

Below the Volume dial is the Voice Control button. The Voice Control button allows you to quickly access Speed, volume, and pitch settings for the TTS and/or media playback. Cycle through the settings with repeated presses of the Voice Control button, and use the Volume dial to adjust the setting.

Below the Voice Control button is the Keylock switch. Place the Keylock switch in the down position to lock the keyboard and in the up position for normal operation.

About halfway between the Keylock switch and the bottom of the unit is a very small pin hole. Inside this hole is located the Reset button. Use a pointed object to press this button for more than 2 seconds to reset the Blaze EZ.

1.4.4 The Top Edge.

The top EDGE of the Blaze EZ contains only a small hole for looping a carrying strap or lanyard.

1.4.5 The Bottom Edge.

If you move around to the edge nearest your body, you will discover 3 ports. From left to right these ports are the headphone jack, the micro USB port, and the stereo microphone jack. Use the headphone jack to connect headphones or external speakers to the unit. Use the microphone jack to connect an external microphone or line-in recording source to the unit. The micro USB port is used to connect the Blaze to a PC for transferring files, to connect a thumb drive or CD-ROM drive to the unit, or to connect the AC adapter for charging the unit.

< The Bottom Edge>

Head phone Jack

Micro USB Port

Stereo micro Jack

1.4.6 The Back Face.

Flip the Blaze over in your hand so that the SD card slot is now on the right side. The back face or underside of the unit contains 2 things. In the top center, directly opposite the Power button on the front face,  is the OCR camera. Below that is a triangular ridged section. If you press down and backward on this section, the majority of the back of the unit slides away to reveal the battery compartment.

2. Getting Started.

This chapter describes basic operations of the Blaze EZ including inserting and removing the battery, powering on and off, charging, and copying content to the unit.

2.1 Inserting and Removing the Battery.

When your Blaze EZ is shipped, the battery is packed separately to prevent any damage during transit. You should insert the battery before using Blaze EZ to ensure normal operation.

As described above, you can access the battery compartment by sliding the back off of the unit. With the keypad facing down and the SD card slot facing to the right, locate the triangular ridged section under the camera on the back of the unit. Press down and backward on the triangular ridge to release and slide the back away from the unit.

To insert the battery pack into the unit, hold it so that the edge with the small ridge is facing toward you with the ridge on the left. There is also a dot on the top surface of the battery, near the bottom left corner. Place the ridged end into the bottom of the battery compartment first. Gently press down on the top of the battery to secure it in place. Slide the back casing on to the unit and click it into place.

To remove the battery pack, slide the back casing off of the unit. Place your fingernail between the top of the battery pack and the edge of the compartment and lift the battery up and out.

2.2 Charging.

This chapter describes how to charge the Battery of the Blaze EZ. When you first receive your Blaze EZ, the battery will not likely contain a full charge. It is recommended that you fully charge the battery before using Blaze EZ.

- Charging using the included AC adapter.

To charge the Blaze EZ using the AC adapter, plug the larger, box-like end into a standard electrical wall outlet. Plug the small, flat end into the micro USB port on the center of the bottom edge of the Blaze EZ. It takes about 3 hours to fully charge the battery.

- Charging using a computer:

You can also charge the Blaze EZ using your computer and the included micro USB cable. Connect the smaller end of the cable to the micro USB port on the center of the bottom edge of the Blaze EZ, and connect the larger end to a USB port on your computer. Charging via a computer usually takes about 4 hours. While the unit is charging, the power LED indicator is red. When charging is complete, the indicator light is green. If the unit is not connected to an AC adapter or computer, the light is off.

2.3 Powering On/Off.

To power on the Blaze EZ, press and hold the Power button for 5 seconds. When you connect the unit to AC power or insert the battery, if it contains a charge, you do not need to press the power button, as booting is automatic. You will hear a short beep to indicate booting has started. In a few seconds, this is followed by a musical flourish with "HIMS" spoken. After this, progress beeps are heard, followed by the power on sound.

Next, the unit announces, "Scanning disk" and the unit scans the internal flashdisk as well as any storage device currently connected for content playable by Blaze EZ. It is not necessary to place content in any set folder structure as this process will scan and index all playable content on all storage devices and make it available for its associated program. After a few seconds, the unit announces "Scanning complete".

When the unit is powered on, you are placed at the Home Menu, described in the next section, and the date and time is announced.

- Note: The date and time will not likely be accurate at the time of purchase. Please see the section on setting the date and time for information on how to adjust it.

To power off Blaze EZ, press and hold the Power button again for 5 seconds. The power down sound is heard, and the unit is powered off.

After the initial boot sequence described above, you can power the unit on and off quickly by holding the “Power” button, and the unit will power on at the Home menu. However, if the battery is depleted, or the unit is reset, the Blaze EZ will go through the more lengthy boot process described above.

2.4 Inserting and Removing the SD CARD.

Locate the long rectangular slot near the bottom of the left edge.

To insert the SD card, locate the corner of the SD card that is cut at a 45 degree angle. Place the SD card so that this cut corner is at the top right when you insert the card into the SD slot. The surface of the label should be face down. Gently press the SD card into the slot until it clicks into place. Do not use excessive force. Only slight pressure is required to insert the SD card in the slot. If the Blaze EZ is turned on, you will hear a sound to confirm that it has been recognized, followed by the announcement, "SD card inserted". The unit then scans the disk for playable content, and returns you to the place on the unit from which you inserted the SD card.

To remove the SD card, gently press the SD card into the card slot until it clicks, then release it. The SD card will pop out of the slot enough for you to grip it and pull it out completely. If the Blaze EZ is turned on, you will hear a sound to confirm that it has detected the removal of the SD card followed by the announcement, "SD card removed".

2.5 Connecting to a computer.

2.5.1 Connecting to a PC.

Locate the small rectangular port on the center of the bottom edge of the Blaze EZ between the headphone and microphone jacks. To transfer files from your PC to your Blaze EZ, turn on the Blaze EZ and connect the small end of the USB cable provided with the unit to the Micro USB port on the bottom edge and connect the larger end to your PC's USB port. The unit announces, "Starting data transfer mode".

Once you connect to your PC, click the "My computer" icon on the PC. The Blaze EZ uses the Media Transfer Protocol connection (MTP), thus, it appears under "Portable Media Devices" with the label "T70". If you press "Enter" on "T70", you can view the content of the "Internal Storage" as well as the content of your SD card if one is inserted. Use normal Windows Explorer commands to copy, cut, paste or delete files as you wish.

- Note: While “Data Transfer Mode” is active the Blaze EZ is not otherwise operational. Pressing keys on Blaze EZ will have no effect. When you disconnect the USB cable from the computer or Blaze EZ, the unit, once again, becomes fully operational.

- Note: if using Windows XP, you must install Windows Media Player 11 to transfer files to the Blaze EZ. Please see Appendix 2 of this manual for instructions on installing Windows media Player 11 with Windows XP.

You may copy and organize documents, media, DAISY books and scanned images using normal Windows commands, in whatever way makes the most sense to you. Blaze EZ will locate playable content no matter where it resides on the disk and index it and make it available for the associated programs. However, the flashdisk (Internal Storage) of Blaze EZ does contain default folders into which content such as Web Radio and Podcast URL's must be placed as well as to help you organize content.

2.5.2 Connecting to a Mac.

As described above, Blaze EZ uses the Media TRANSFER  Protocol, (MTP) connection for transferring data to and from a computer. As the Mac operating system does not natively support this connection type, it is necessary to download a third-party executable which will allow you to drag and drop files into Blaze EZ. You may download an appropriate executable at the following link:

http://www.android.com/filetransfer/

Once you have installed this small application, connect your Blaze EZ to your Mac and power it on. The Blaze EZ will automatically launch the File Transfer application, and you can use VoiceOver to drag and drop files into the Blaze EZ, again, under the label "T70".

For information on how to drag and drop items using VoiceOver, visit the following link:

http://help.apple.com/voiceover/info/guide/10.7/English.lproj/_1124.html

2.6 Folders used in the Blaze EZ.

As previously explained, it is not necessary to place content playable by the Blaze EZ in specific folders. When the internal storage and/or SD card are scanned, all playable content is recognized and made available to the programs in which it can be played. For example, When you press the "Explorer" button in the Media player, all media content is available no matter where it is placed, and no matter on which drive it is contained. However, there are a number of folders included on the Blaze EZ with unique characteristics and special purposes.

    - OCR: Optical character recognition image or text files are stored in the "OCR" folder under the following sub-folders:

- Image: Images captured using the built-in camera are stored in this folder.

- Import: Images can be copied from another device to this folder and can be recognized by Blaze EZ.

- Temporary: Files that are created during capturing and character recognition are stored in temporary files in this  folder.

- Text : Recognition result files are stored in this folder.

    - Podcast: Podcast content and address information is stored in this folder, including  URL, XML, OPML. To subscribe to podcasts on the Blaze EZ, you must copy the podcast address information into this folder.

    - Radio: Files recorded from the FM Radio are saved in this folder.

    - Record: Voice recordings and line-in recordings are saved in this folder.

    - Web radio: Web addresses of Internet streaming broadcasts must be located here to be recognized by the Web Radio application, including M3u, and PLS playlist addresses.

3. Basic Functions.

3.1 The Home Menu.

When you power on the unit for the first time, you are placed in the Home Menu. You can also get to the Home Menu by pressing Cancel when in a program. The Home Menu contains a combination of things including status information, settings, and executable programs not accessible via the program buttons. You can navigate among the items in the Home Menu using the Left and Right arrows. Press the Select button to activate the focused item in the menu.

The Home Menu contains the following items:

- Date and Time information: the current date and time is the first item in the Home menu. When the Blaze EZ boots, it is always placed in the Home menu on the Date and Time item.    Press the "Select" button to open the "Set Date and Time" dialog box.

- Power Status: announces the percentage of battery charge, whether it is Currently plugged into the charger, and whether the unit is currently running on battery or AC power.

- System Information: Shows the current firmware version and displays the amount of free space on the flash memory. Press the 'Select' button to access more detailed system information including the product serial number, total space on the SD card and the Mac address of the unit.

- Guide Voice Settings: Press "Select" to open this dialog wherein you can set the guide voice speed, volume, pitch and gender.

- Record Settings: Press "Select" to open this dialog box in which you can set recording quality, microphone sensitivity, and the recording source.

- Bluetooth Information: this item announces whether Bluetooth is on or off, and if on, whether there are any active connections. Press the "Select" button to open the "Bluetooth Connection Settings" dialog box.

- Wireless LAN Information: Announces whether Wi-fi is on or off, and, if on, if the Blaze EZ is connected to any networks. Press the  "Select" button to open the "Wireless Settings" dialog box.

- Web Radio: Press "Select" on this program to listen to web radio stations from around the world.

- Podcasts: Press "Select" on this program to download and play podcasts you are subscribed to.

- Library Services: Press “Select” to open a menu containing available search and download applications for electronic libraries for the blind supported in the country in which you live.

- Utilities: Press "Select" to access the Utilities menu, which gives you access to utilities related to disk scanning, upgrading the unit and CD-ROM playback.

- Options: press “Select” to set options related to language for reading and OCR.

3.2 Adjusting Volume and other properties.

You can use the Voice Control button and the Volume dial on the right side of Blaze EZ to adjust the volume of media playback, the guide voice, and the reading voice. You can also use it to change various other settings, such as speed, and media preamp.

To adjust a setting, press the Voice Control button just below the Volume dial repeatedly until you reach your desired setting. Then, use the Volume dial to increase or decrease the value of that setting.

- ·Note: the default setting is always the master volume, and the volume dial returns to controlling that value after a few seconds of inactivity. Pressing any other key on the unit also returns the Volume dial to controlling the main volume.

The available settings differ depending on your location in Blaze EZ. The following settings are available in their respective locations on Blaze EZ.

- From the Home Menu: Guide Voice Volume, Guide Voice Speed and Guide Voice Pitch.

- From the Media Player: Media Volume, Media Preamp, Media Playback Speed and Media Equalizer.

- During DAISY text playback: Reading Voice Volume, Reading Voice Speed, and Reading Voice Pitch.

- During DAISY Audio Playback: DAISY Audio Volume, DAISY Audio Preamp, and DAISY Audio Speed.

3.3 Adjusting Voice Settings.

This section describes how to adjust the volume, speed and pitch of the speech of the main voice which guides you through the unit and announces system messages, etc.

The Guide Voice Settings are located in the Utilities Menu. From the Home menu, use the Left or Right arrow to navigate to the Utilities menu, and press “Select”. Use the Left and Right arrows to navigate to "Guide Voice Settings", and press the Select button to open the dialog.

Use the Up and Down arrows to move between the settings in the dialog. Use the Left and Right arrows to change the value of the setting. Press "Select" to save the settings, or press "Cancel"  to cancel saving the settings and exit the Voice Settings dialog box. The Guide Voice Settings dialog consists of 5 items:

- Guide Voice Speed: Adjusts the speed of the voice guide. Use the Left and Right arrows to select a value from 1 to 15.

- Guide Voice Pitch: Adjusts how high or low the voice sounds. Use the Left and Right arrows to select a value from 1 to 15.

- Guide voice volume:  Adjusts how loudly the guide voice speaks. Use the Left and Right arrows to choose a value from 1 to 15.

- Voice Type: Choose to use a male or female voice. Use the Left and Right arrows to move between the options.

- Upper Case Alert: Choose whether to have the speech alert you of capital letters. Use the Left and  Right arrows to choose between On or Off.

3.4 Checking Date  and Time.

You can check the date and time on the Blaze EZ in two ways:

- Press the Power button quickly. If you press the Power button quickly from anywhere on the unit, the date and time is announced.

You can even use this method when the power is off. However, it should be noted, when the power is off, it takes a couple of seconds for the date and time to be announced.

- Bring up the Home Menu. The Date and time is the first item in the Home Menu. Any time you exit a program and return to the Home Menu, the date and time is announced as the first status item.

3.5 Setting the Date and Time.

You can set the Date and Time in 2 ways: you can set it manually using the Date and Time dialog, or you can have it automatically synchronized with a time server.

3.5.1 Manually setting the Date and Time.

Use the following steps to manually set the Date and Time:

- From the Home menu, press the Left or Right arrow to navigate to the Date and Time.

- Press the "Select" button to open the "Date and Time" dialog.

- You are placed on the "Month". Use the Left and Right arrows to set the month.

- Press the Down arrow, and use the Left and Right arrows to set the day.

- Press the Down arrow, and use the Left and Right arrows to set the year.

- Press the Down arrow, and use the Left and Right arrows to choose the time format between 12 and 24 hour:

- Press the Down arrow, and use the Left and Right arrows to choose AM or PM.

· Note: If you choose the 24 hour time format, this item is not displayed.

- Press the Down arrow, and use the Left and Right arrows to set the hour.

- Press the Down arrow, and use the Left and Right arrows to set the minute.

- Press the Down arrow, and use the Left and Right arrows to set the time zone.

- Press the Down arrow, and use the Left and Right arrows to choose whether or not to use Daylight Savings time.

- When you have finished choosing all your settings, press the "Select" button to save your settings, and you are returned to the Home menu. If you do not want to save your settings, press the "Cancel" button, and you are returned to the Home menu without modifying the Date and Time.

3.5.2 Synchronizing with a Time Server.

To set the Date and Time by Synchronizing with a time server, follow these steps:

- From the Home menu, use the Left and Right arrows to navigate to the Date and Time.

- Press the "Select" button to open the "Date and Time" dialog.

- Use the Up or Down arrow to navigate to "Synchronization". Press the Left or Right arrow to navigate to "Yes".

- Press the "Select" button and the time is synchronized with a time server, and you are returned to the Home menu.

3.6 Checking Power Status.

To check the power status of Blaze EZ, from the Home menu, press the Right arrow to get to the power status item. The Power Status gives you the following information:

- Type of power being used: battery or AC adapter.

- Percentage of battery charge.

- Whether the battery is currently charging.

3.7 Setting the Sleep Timer.

The Sleep Timer allows you to choose a specified amount of time after which the Blaze EZ will automatically power off  while reading a book, listening to music, etc. To set the Sleep Timer, follow these steps:

- Press and hold the "Select" button for more than 2 seconds. The "Sleep Timer" dialog box is displayed.

- Press the Left or Right arrow to set the desired time. The time can be set to zero (off), 3minutes, 5 Minutes, 10 minutes, 15 minutes, 20 minutes, 30 minutes, 45 minutes, 60 minutes, 90 minutes, 120 minutes, or 180 minutes.

- Press the "Select" button, and the setting is saved and you are returned to the position from which you initiated the Sleep Timer setting. If you want to cancel setting the Sleep Timer, press the "Cancel" button. Setting is canceled, and you are returned to the position from which you executed the Sleep Timer.

3.8 Checking System Information.

To access basic system information about your Blaze EZ, from the Home menu, press the Right arrow until you hear the software version and free space announced. Press the "Select" button to open the "System Information" dialog box. The "System Information" dialog displays the following information:

- Firmware version.

- Build date.

- Serial number.

- Total space and remaining space of the internal flashdisk.

- Total space and remaining space on the SD card. (Only appears if an SD card is inserted.)

- Mac Address (only appears after you have connected to Wi-fi).

- Open Source License: You can press select on this option to view the open source license information for Blaze.

To move among the items in the dialog box, press the Up and Down arrows. Press the Left and Right arrows to navigate the information character  by character. When you have finished reviewing the system information, press  the "Cancel" button to close the dialog box.

4. Using the Media Player.

4.1 Supported Media Formats.

The Media Player on the Blaze EZ supports the following media formats:

- MP3 320 Kbps, 44100 Hz, Stereo

- MP4-AAC, M4a)

- Wave 16bit 48000Hz

- Ogg Vorbis

- ASF ASF

- FLAC Q1 & Q10

- Avi

- WMV WMV

You can use the Media Player to play music, books, or podcasts stored on the Blaze EZ, an SD card or USB flash drive. You can begin playing media content immediately with the touch of just 1 button.

4.2 Playing Audio Files.

To start media playback, simply press the left-most button on the row above the arrow keys labeled with an M in Braille. If you have not previously used the Media Player, Blaze EZ begins playing the first media file on the unit. If you have used the Media Player previously, playback resumes at the last file you played.

To close the Media Player, press the "Cancel" button, and the media Player closes and you are returned to the Home menu.

4.3 Play and Pause.

To pause playback, press the M button briefly while media is playing. Press the M button briefly once again to resume playback.

4.4 Moving by Track.

While the Media Player is open, you can move back and forth by track using the Left and Right arrows. Press the Right arrow to move to the next track, and use the Left arrow to move to the previous track.

4.5 Moving by Time.

To move through audio by time, press and hold the Left or Right arrow. Hold the Right arrow to move forward, and hold the Left arrow to move backward. As you hold the Left or Right arrow, the guide voice announces time elements by which you can move. The longer you hold the arrow key, the larger the amount of time. When your desired amount of time is announced, release the arrow key, and you are moved backward or forward by that amount of time.

For example, if you want to move backward 20 seconds, press and hold the Left arrow. You will hear, "5 seconds, 10 seconds, 15 seconds, 20 seconds". When the voice announces "20 seconds", release the arrow key, and you are moved backward 20 seconds in your audio file.

4.6 Changing Navigation units.

You can actually use the Left and Right arrows to move by several units of navigation. To adjust the unit of movement of the Left and Right arrows, use the Up and Down arrows. By default, the Left and Right arrow keys will move back and forth by one track. However, if you press the Up or Down Arrow, you can set them to move by5 tracks, 10 tracks, to the first or last track, to the beginning or end of the track, 10 minutes, 3 minutes, 30 seconds, 5 seconds, or mark. The "Mark" item only appears if marks are present in the file, and allows you to use the Left and Right arrows to navigate between them.

So, to navigate your current playlist  by3 minute segments, press the Up or Down arrow until the unit announces "3 minutes". Then, use the Left and Right arrows to navigate back and forward by3 minute segments.

4.7 The menu.

The menu in the Media player contains settings related to media playback, as well as additional navigation options. You can open the menu by pressing the “Select” key. Use the Up and Down arrows to move among the options, and press the “Select” key to open your desired menu item.

The media Player’s menu includes the following:

- Move by Time

- Move by percent

- Move by Number

- Mark Manager

- Equalizer Settings

- Media Settings

- Content Information

Marks are covered in section 4.8 and the Equalizer Settings are covered in section 4.9 of this manual. The remaining sections are covered below.

4.7.1 Move to a specified time.

To move to a specific time, press”Select” to open the menu, then press “Select” again to open the dialog, as it is the first item in the menu. The elapsed and total times are announced, and you are placed in an edit box in to which you can enter the specific time you want to move to. Enter the time in 6 digits: 2 digits for the hour, 2 digits for the minute, and 2 digits for the second. Thus, to move to 5 minutes and 20 seconds in the current track, enter 000520. You can enter and review the text in the Edit box using the same method as you do to enter a Wi-fi key or other text on the unit.

When you have finished entering the time to move to, press and hold the “Select” key, and playback resumes at the time you specified.

4.7.2 Moving by percentage.

To move to a specific position in the file according to percentage, press the “Select” key to open the menu, arrow down to “Move By percent”, and press “Select” again. The current percentage is announced, and you are placed in an edit box in to which you are prompted to type the percentage to move to. Use the usual text entry method to enter the percentage of the file you want to move to and press and hold the “”Select” key. Playback resumes at the percentage of the file you specified.

4.7.3 Moving to a specified track.

To move to a specific track in the playlist, press the “Select” key to open the menu, arrow down to “Move by Number”, and press “Select” again. The current track number and the total number of tracks are announced, and you are placed in an Edit box in to which you are prompted to type the number of the track you want to move to. Use the usual text entry method to enter the number of the track you want to move to and press and hold the “Select” key. The Media player begins playing the track you specified.

4.7.4 Setting Media preferences

To view or change the Media Settings, open the Media Player menu, arrow down to “Media Settings” and press “Select”.

Use the Up and Down arrows to move among the various settings in the dialog. Use the Left and Right arrows to adjust the settings.

The Media Settings include the following;

- Announce File Name: use the Left and Right arrows to choose between On and Off.

- Set Count for Repeat Highlight Mark: use the Left and Right arrows to choose among Off, from 1 to 10 times, and Loop.

- Repeat Setting: Use the Left and Right arrows to choose among, off, Repeat File, and Repeat All.

- Save bookmark when exiting Media: Use the Left and Right arrows to choose among  after 5 minutes, 10 minutes, 30 minutes, always  and Off.

- Press the “Select” key to save your settings. Press “Cancel” to exit the Media Setting without saving your changes.

4.7.5 Getting Playback Information.

You can obtain information on the currently playing file by holding the “Explorer” key.

- File Name: announces the current file name and format.

- Play Time: the elapsed playing time of the current file.

- Total Playing Time: the total playing time of the current file.

- Mark information: announces the number of marks in the current file. If there are no marks, no mark information is announced.

- Current track number: announces the track number according to the current playlist.

- Total track number: announces the total number of tracks in the current playlist.

- Status: announces Play or Pause status.

The information dialog shows the information for the current file in list form. Use the Up and Down arrows to navigate among the various pieces of information. Use the Left and Right arrows to review the current piece of information character by character.

To exit the “Information” dialog press the “Cancel” button.

You can also open the “Information” dialog from the Media Player menu.

4.8 Marks.

In longer audio files, such as recorded lectures or books, or files you intend to use for  teaching, etc, it can be useful to mark certain sections so that you can easily return to them later. You can identify such sections using bookmarks, Voice marks, or highlight marks.

You can place up to 999 marks in each file, and you may not duplicate mark numbers, even if the marks are of a different type. For example, you may  not have a bookmark and a highlight mark both labeled as number 5. Each mark, no matter the type, will have a unique number from 1-999.

4.8.1 Inserting a Bookmark.

To insert a mark, follow these steps:

- Locate the place in the content where you want to insert the mark, and press the "Select" button.

- Use the Up and Down arrows to move to the “Mark Manager, and press “select”.

- The Mark Manager menu is displayed, and you are placed on the first item, "Insert bookmark".

- Press the "Select" button again.

- You are prompted for the number of the mark to insert.

- Use the Left or Right arrow to move to your desired number for the mark. Use the Up and Down arrows to move in groups of 10. For example, if you want to set the current mark as number 23, and the current selection is 1, press the Right arrow  twice to get to number  3, then press  the Up arrow twice to move to 13, then 23.

- Press the "Select" button to save the mark.

If the mark already exists, you  are prompted whether to move or delete the existing mark. Use the  Left and Right arrows to move among the choices, and press "Select" to activate your choice.

- Press "Cancel" at any time to cancel the setting of the mark.

4.8.2 Inserting a Voice Mark.

Setting a Voice Mark allows you to attach a voice  recording to the mark to provide additional information or an anotation to the content you  are marking.

Use the following steps to insert a voice mark:

- Locate the place in the content which you want to mark, and press "Select".

- Use the Up and Down arrows to navigate to the Mark Manager, and press “Select”.

- The Mark Manager is opened, and  the first item "Bookmark" is focused.

- Press the Down arrow to navigate to "Voice Mark" and press "Select".

- The unit prompts, "Start recording", and you have up to 40 seconds to record your audio anotation.

- When you have finished recording, press the "Select" button.

- You are prompted to choose the number  for the voice mark.

- Use the Left or Right arrow to choose the number for the mark. Use the Up or Down arrows to move by groups of 10 numbers.

- Press "Select" to save the mark. If a mark already exists under that number, you are asked whether you want to delete or move the existing mark. Use the Left or Right arrow to move among the choices, and press "Select" to activate your desired option.

- Press "Cancel" at any time to cancel the setting of the mark.

4.8.3 Inserting a Highlight Mark.

There may be times you wish to highlight a certain section of  content, similar to underlining a passage of text. You can do this by setting a highlight mark.

Use the following steps to set a highlight mark:

- Locate the place in the content which you want to mark, and press "Select".

- Use the Up and Down arrows to navigate to the Mark Manager and press “Select”.

- The Mark Manager is opened, and  the first item "Bookmark" is focused.

- Press the Down arrow to navigate to "Highlight Mark" and press "Select".

- The start position is marked at your current position, and playback continues.

- When playback reaches the point at which you want to end the highlight mark, press "Select" again.

- You are prompted to choose the number  for the highlight mark.

- Use the Left or Right arrow to choose the number for the mark. Use the Up or Down arrows to move by groups of 10 numbers.

- Press "Select" to save the mark. If a mark already exists under that number, you are asked whether you want to delete or move the existing mark. Use the Left or Right arrow to move among the choices, and press "Select" to activate your desired option.

- Press "Cancel" at any time to cancel the setting of the mark.

4.8.4 Moving to a Mark.

You can move to an existing mark in 2 ways: either by using the "mark" navigation unit available during playback, or by specifying the mark to move to using the Mark Manager. If you have few  marks in a file, or you wish to move through  your marks sequencially, using the "mark" movement unit may be most efficient. However, if you have many marks in a file, and wish to quickly access a specific marked position, you may find it more advantageous to use the Mark Manager.

Use the following steps to move to a mark using the "mark" navigation unit:

- Use the Up or  Down arrow to choose "Mark" as the unit to move by.

- Use the Left and Right arrows to move among the various marks in the file.

Use the following steps to move to a mark using the Mark Manager:

- At any time during playback, press "Select" to open the menu.

- Move to the Mark Manager with the Up or Down arrow, and press “Select”.

- The Mark Manager is opened, and  the first item "Bookmark" is focused.

- Press the Down arrow to navigate to "Go to Mark" and press "Select".

- You are prompted to choose the number  of the mark to move to.

- Use the Left or Right arrow to choose the number for the mark. Use the Up or Down arrows to move by groups of 10 numbers.

- Press "Select" and playback begins at the marked position.

Press "Cancel" at any time to cancel moving to a mark.

4.8.5 Deleting a Mark.

Use the following steps to delete a mark:

- At any time during playback, press "Select" to open the menu.

- Move to the Mark Manager with the Up or Down arrow, and press “Select”.

- The Mark Manager is opened, and  the first item "Bookmark" is focused.

- Press the Down arrow to navigate to "Delete Mark" and press "Select".

- You are prompted to choose the number  of the mark to delete.

- Use the Left or Right arrow to choose the number for the mark. Use the Up or Down arrows to move by groups of 10 numbers.

- Press "Select" and the mark is deleted.

- Press "Cancel" at any time to cancel deleting a mark.

4.9 Adjusting Volume, speed, and Equalizer.

You can use the Volume dial to adjust the media volume, preamp, playback speed, and equalizer.

- Media Playback Volume: You can adjust media playback volume using values 1-15.

- Preamp: audio recordings are often at different volume levels. You can adjust the preamplifier level for each file to compensate for this variation. The default value is 0. You  can adjust the preamp level using values from — -4 to 6.

- Media Rate: Adjust the playback speed using values from 1 to 15.

- Equalizer: Depending on the type of audio  you are playing, you may wish to change the characteristics of the audio sound using the Equalizer setting. The available  options are: Auto, Normal, Classical, Dance, Jazz, Pop and Rock.

Use the "Voice Control" button to determine which setting the Volume dial modifies, and turn the Volume dial toward the top of the unit to increase the value, and toward the bottom of the unit to decrease it. For example, to change the playback speed, press the "Voice Control" button twice to get to "Media rate". Then use the Volume dial to increase or  decrease the rate of media playback.

4.10 Using The Explorer.

If you wish to locate a certain song, album, or podcast file, you can use the "Explorer" to easily search for and navigate media content stored on your Blaze EZ.

As folders and files appear in easily navigable lists, you can locate podcasts, books, music, or any other media files stored on your unit. You can also easily delete files without connecting to a PC.

To access files and folders containing media content on your Blaze EZ, press the "Explorer" button while the Media Player is running. The Explorer" button is a dot-like button on the bottom row of keys between the "Cancel" button and the "OCR" button. By default, the Explorer shows only folders containing media content. However, if you set the Explorer Mode to “Directory Mode” in the Global Settings, you may browse the files and folders as they are stored.

Use the following keystrokes in the Explorer:

- Use the Up and Down arrows to move through a file or folder list.

- Use the Left arrow to move back a level in the folder structure.

- Use the Right arrow to move forward a level in the file/folder structure.

- Use the "Select" button to play the focused file or folder.

- Press and hold the "Explorer" button to get the path for the current item.

- Press and hold the "Cancel" button for more than 2 seconds to delete the focused file or folder. You are prompted whether you are sure you wish to delete. Press "Select" for yes, or "Cancel" for no. You can also press and hold the "Cancel" button during playback to delete the current file without entering the "Explorer".

You can select multiple items for playback and other actions by pressing the Media key. Press and hold the Media key to cycle among additional selection options: “Select All”, “Select from current to end”, “Select from current to beginning”, and “Start continuous Selection”.

You can access a menu of file management options by pressing the Explorer key once more while the Explorer is opened.

Again. Use the Up and Down arrows to choose from “Move By” (number), “Copy”, “Cut”, and “Paste” (if items are on the clipboard). Press “Select” to activate your desired action on the selected files.

5. FM Radio.

In this section you will learn how to play the FM radio, how to manually and auto scan, how to set presets, and how  to record the radio output.

The FM Radio can access local stations at frequencies from 87.5MHz to 108MHz.

- Note: In order to receive a radio signal, you must connect a headphone cable or audio patch cord for use as an antenna. Please connect headphones or a patch cord to the headphone jack on the left end of the bottom EDGE of blaze EZ.

5.1 Playing the Radio.

To play the FM radio, simply press the "Radio" button, labeled with R in Braille and located in the center of the row of buttons just above the arrow keys.

If you do not receive any radio stations, consider the following:

- Make sure you have headphones or an audio patch cord connected to the headphone jack on Blaze EZ.

- You may not be placed on a frequency for which there is a local radio station. Use the Left and Right arrows to adjust the frequency until you are able to find a station.

- The characteristics of your location may not be optimal for receiving good radio reception.

To mute the radio, press the R button quickly. Press it once again to resume radio playback. Press the "Cancel" button to exit the FM Radio and return to the Home menu.

5.2 Adjusting Radio Volume.

Use the Volume dial on the right edge of the unit to adjust the radio volume. Turn it toward the top of the unit to increase the volume, and turn it toward the bottom of the unit to decrease it.

5.3 Navigating Frequencies.

Use the Left and Right arrows to adjust frequency backward and forward by0.1MHz. As stated above, the available frequency range is 87.5MHz to 108MHz.

To automatically detect stations press and hold the Left or Right arrow to auto scan backward and forward respectively.

5.4 Selecting and Moving to Channels.

You may find it cumbersome to manually move through all the available channels, especially if you have specific stations you know you like to listen to. You can easily save presets, simply by pressing "Select" while playing the radio frequency you want to save. You can save up to 30 presets.

You can also choose to auto register all radio stations in your area. To do this, while the FM Radio is open, press and hold the "Explorer" button for 2 seconds. Blaze EZ searches for all available radio stations in the area, and automatically registers them as presets.

You can increase the sensitivity of the channel search by pressing and holding the Down arrow. However, it should be noted, that increasing the sensitivity may result in the registering of many stations that do not come in clearly.

You can use the Up and Down arrows to navigate between your preset channels. When you reach the last registered channel, if you continue to navigate with the Up or Down arrow, you will wrap around to the first channel in the list.

You can also navigate directly to a specific preset channel using the "Explorer" button. When you press the "Explorer" button, all of your preset channels are presented in a list. Use the Up and Down arrows to navigate to your desired channel, and press the "Select" button to go to that channel.

To delete a preset, navigate to it, and press and hold the "Cancel" button.

5.5 Using the Internal Speaker.

Listening to the radio via headphones for great lengths of time can become uncomfortable. You can listen to the radio via the internal speakers, even though the headphones must be connected as an antenna. To hear the radio through the internal speaker of Blaze EZ, press and hold the "R" button. The radio will output through both the headphones and the internal speakers. Press and hold the "R" button again to return the output to the headphones only.

5.6 Recording a Radio Broadcast.

To Record the current radio frequency, press and hold the "Record" button for 2 seconds. Depending how you have set your Record Settings, you are prompted through beep or  voice notification that the recording has started.

To pause the recording, press the  "Record" button quickly.  Briefly press the "Record" button again to resume recording. To stop and save the recording, press the "Select" button. You are notified that the recording is complete using the method set in the "Record Settings" dialog.

To cancel the recording, press the "Radio" button or the "Cancel" button. You are asked whether you  want to cancel the recording. Press "Select" to cancel recording, or press "Cancel" to resume recording.

Recordings are saved to the "Radio" folder on the flashdisk of Blaze EZ and are named with the frequency and the date and time. Recording quality settings are the same as those set for voice recording. See the section of this  manual on the "record Settings" dialog  for more details.

6. Book Reader.

The Book Reader program on Blaze EZ is able to play DAISY books as well as several document file types.

6.1 Supported File Types.

The Book Reader supports the following formats:

Document formats:

- Plain text document: TXT.

- Rich Text Format: RTF.

- MS Word document: DOC/DOCX.

- Markup language document: HTM, HTML, XML.

- Sense notetaker document: HBL HPF.

- e-book: EPUB.

- Adobe Acrobat document: PDF.

- Note: Only text-based PDF documents are supported in the Book Reader. PDF's created from images cannot be supported.

DAISY formats:

- DAISY 2.0/2.02/3.0, DAISY XML / Digital Talking Book.

- ANSI / NISO Z39.86 2002, 2005.

- NIMAS 1.1,

- National Library Service (NLS).

- Content from BookShare.org.

- Learning Ally

- Open Library/Archive.org.

6.2 Play and pause.

To play content in the Book Reader, simply press the "Book" button, labeled with a B in Braille and located on the right of the row of buttons just above the arrow keys.

If you have not played book content previously, the Book reader begins playing the first available book or document. If you have played content previously, Blaze EZ resumes playing the book or document you  were last reading at the position at which you exited it.

Press the "Book" button briefly to pause playback. Press it once again to resume playing the book or document. Press the "Cancel" button to exit book or document playback and return to the Home menu. When you exit the Book Reader, the position at which you exited is marked in the book or document and playback resumes at that position the next time you open that book or document.

6.3 Navigation.

You can navigate books and documents in the book reader in several ways, including headings, paragraphs, sentences, words, time, etc. Press the Up or Down arrow to select the movement unit. Press the Left or Right arrow to move back and forward by the chosen movement unit.

The movement units available differ according to the type of content being played. Information on the movement units available for the various content types is as follows:

- Content: When this item is selected, pressing the Left and Right arrows moves you among the books and documents available on the BLAZE EZ.

- Heading: This item appears only when playing DAISY content. When this item is selected, using the Left and Right arrows navigates back and forward by the headings in the DAISY book.

- Level: This item appears only during DAISY playback. Available level movement is dependent on how the DAISY book has been configured. It may contain levels from 1-6.

- Phrase: This item also appears only during DAISY playback. Phrases are generally defined as the amount of text spoken in 1 breath, however, there is no set standard. NLS books adapted from cassette tape have been known to use the phrase mark up to represent an entire cassette track. When this item is selected the Left and Right arrows move to the previous and next phrase respectively.

- Page: When this movement unit is selected, the Left and Right arrows will move to the previous and next page respectively.

- Paragraph: When this item is selected, the Left and Right arrows move to the previous and next paragraph respectively.

- Sentence: When this item is selected the Left and Right arrows move to the previous and next sentence. This item appears only in documents and text-based DAISY content.

- Line: The Left and Right arrows move to the previous and next line. This item appears only during playback of documents and text-based DAISY content.

- Word: When this item is selected, the Left and Right arrows move to the previous and next word. This item appears only during playback of documents or text-based DAISY content.

- Character: The Left and Right arrows navigate to the previous and next character. Appears only during playback of documents or text-based DAISY content.

- Time movement: 5 seconds, 30 seconds, 3 minutes, and 10 minutes. Use the Up and Down arrows to select the time movement unit, and the Left and Right arrows to move back and forward by the selected time. Time movement units are available only during playback of DAISY audio content.

- Beginning or End: The Left arrow navigates to the beginning of the book or file, and the Right arrow navigates to the end of the book or file.

6.4 Quick Movement.

You may also use quick movement methods to navigate through a document or book more quickly by pressing and holding the Left or Right arrow.

The movement method will vary based on the type  of content you are playing. For example, if you are playing DAISY audio, pressing and holding the Right arrow, will give you options like 30 seconds, 1 minute, 3 minutes, etc. If you are playing a text-based DAISY book or document, holding the Right arrow gives you  movement options like 5 sentences, 10 sentences, 15 sentences, etc. The longer you hold the arrow key, the greater the amount of movement. When you hear the movement unit you want, release the  arrow key, and you are taken to the place in your book or document according to the movement type and amount you chose.

6.5 Marks.

In longer files, such as books, or files you intend to use for  teaching, etc, it can be useful to mark certain sections so that you can easily return to them later. You can identify such sections using bookmarks, Voice marks, or highlight marks.

You can place up to 1000 marks in each file, and you may not duplicate mark numbers, even if the marks are of a different type. For example, you may  not have a bookmark and a highlight mark both labeled as number 5. Each mark, no matter the type, will have a unique number from 1-999.

6.5.1 Inserting a Mark.

To insert a mark, follow these steps:

- Locate the place in the content where you want to insert the mark, and press the "Select" button to open the menu.

- Arrow down to the Mark Manager, and press “Select” again.

- The Mark Manager menu is displayed, and you are placed on the first item, "Insert bookmark".

- Press the "Select" button again.

- You are prompted for the number of the mark to insert.

- Use the Left or Right arrow to move to your desired number for the mark. Use the Up and Down arrows to move in groups of 10. For example, if you want to set the current mark as number 23, and the current selection is 1, press the Right arrow  twice to get to number  3, then press  the Up arrow twice to move to 13, then 23.

- Press the "Select" button to save the mark.

If the mark already exists, you  will be prompted whether to move or delete the existing mark. Use the  Left and Right arrows to move among the choices, and press "Select" to activate your choice.

Press "Cancel" at any time to cancel the setting of the mark.

6.5.2 Inserting a Voice Mark.

Setting a Voice Mark allows you to attach a voice  recording to the mark to provide additional information or an anotation to the content you  are marking.

Use the following steps to insert a voice mark:

- Locate the place in the content which you want to mark, and press "Select" to open the menu.

- Arrow down to the Mark Manager and press “Select” again.

- The Mark Manager is opened, and  the first item "Bookmark" is focused.

- Press the Down arrow to navigate to "Voice Mark" and press "Select".

- The unit prompts, "Start recording", and you have up to 40 seconds to record your audio anotation.

- When you have finished recording, press the "Select" button.

- You are prompted to choose the number  for the voice mark.

- Use the Left or Right arrow to choose the number for the mark. Use the Up or Down arrows to move by groups of 10 numbers.

- Press "Select" to save the mark. If a mark already exists under that number, you are asked whether you want to delete or move the existing mark. Use the Left or Right arrow to move among the choices, and press "Select" to activate your desired option.

Press "Cancel" at any time to cancel the setting of the mark.

6.5.3 Inserting a Highlight Mark.

There may be times you wish to highlight a certain section of  content, similar to underlining a passage of text. You can do this by setting a highlight mark.

Use the following steps to set a highlight mark:

- Locate the place in the content which you want to mark, and press "Select" to open the menu.

- Arrow down to the Mark Manager, and press “Select” again.

- The Mark Manager is opened, and  the first item "Bookmark" is focused.

- Press the Down arrow to navigate to "Highlight Mark" and press "Select".

- The start position is marked at your current position, and playback continues.

- When playback reaches the point at which you want to end the highlight mark, press "Select" again.

- You are prompted to choose the number  for the highlight mark.

- Use the Left or Right arrow to choose the number for the mark. Use the Up or Down arrows to move by groups of 10 numbers.

- Press "Select" to save the mark. If a mark already exists under that number, you are asked whether you want to delete or move the existing mark. Use the Left or Right arrow to move among the choices, and press "Select" to activate your desired option.

- Press "Cancel" at any time to cancel the setting of the mark.

6.5.4 Moving to a Mark.

You can move to an existing mark in 2 ways: either by using the "mark" movement unit available during playback, or by specifying the mark to move to using the Mark Manager. If you have few  marks in a file, or you wish to move through  your marks sequencially, using the "mark" movement unit may be most efficient. However, if you have many marks in a file, and wish to quickly access a specific marked position, you may find it more advantageous to use the Mark Manager.

Use the following steps to move to a mark using the "mark" movement unit:

- Use the Up or  Down arrow to choose "Mark" as the unit to move by.

- Use the Left and Right arrows to move among the various marks in the file.

Use the following steps to move to a mark using the Mark Manager:

- At any time during playback, press "Select" to open the menu, arrow down to Mark Manager, and press “select”.

- The Mark Manager is opened, and  the first item "Bookmark" is focused.

- Press the Down arrow to navigate to "Go to Mark" and press "Select".

- You are prompted to choose the number  of the mark to move to.

- Use the Left or Right arrow to choose the number for the mark. Use the Up or Down arrows to move by groups of 10 numbers.

- Press "Select" and playback will begin at the marked position.

- Press "Cancel" at any time to cancel moving to a mark.

6.5.5 Deleting a Mark.

Use the following steps to delete a mark:

- At any time during playback, press "Select" to open the menu, navigate to the Mark Manager, and press “Select” again.

- The Mark Manager is opened, and  the first item "Bookmark" is focused.

- Press the Down arrow to navigate to "Delete Mark" and press "Select".

- You are prompted to choose the number  of the mark to delete.

- Use the Left or Right arrow to choose the number for the mark. Use the Up or Down arrows to move by groups of 10 numbers.

- Press "Select" and the mark is deleted.

- Press "Cancel" at any time to cancel deleting a mark.

6.6 Adjusting Volume and Speed

You can use the Volume dial to adjust the reading voice volume, audio preamp, playback speed, and voice pitch. Use the "Voice Control" button to cycle through the available adjustments. The adjustable settings will vary depending on the type of content being  played. For example, an audio DAISY book will contain the "Preamp" option, but will not contain the reading Voice option as this applies to the TTS for text playback.

- Reading Voice Volume: You can adjust Reading Voice volume using values 1-15. Appears during document or DAISY text playback.

- Reading VoicePitch: You can adjust the Reading Voice pitch using values 1-15. Appears during document or DAISY text playback.

- Preamp: audio recordings are often at different volume levels. You can adjust the preamplifier level for each file to compensate for this variation. The default value is 0. You  can adjust the preamp level using values from — 4 to 6. Appears only during DAISY audio playback.

- Playback speed/Text-to-speech rate: Adjust the playback speed using values from 1 to 15. Is available for both DAISY audio and text/document playback.

Use the "Voice Control" button to determine which setting the Volume dial will modify, and turn the Volume dial toward the top of the unit to increase the value, and toward the bottom of the unit to decrease it. For example, to change the playback speed, press the "Voice Control" button twice to get to playback speed. Then use the Volume dial to increase or  decrease the speed of playback.

6.7 Switching Daisy Modes.

Some DAISY books contain both text and audio. In this case, you can switch between playback of the text and the audio.

While a DAISY book containing both text and audio is open, press and hold the "Book" button. Each time you press and hold the "Book" button, it toggles between text and audio mode.

6.8 Deleting Content.

You can easily  delete the currently playing file or book during playback. To do this, press and hold the "Cancel" button for more than 5 seconds. You are asked if you want to delete the content. Press "Select" for yes, or press "Cancel" to cancel the deletion.

6.9 The Menu.

The Book Reader menu contains various settings and additional navigation options. Items will differ slightly depending on the type of information you are reading (DAISY audio, DAISY text, or document files).

Press the “Select” key when the Book Reader is running to open the Book Reader menu. Use the Up and Down arrows to navigate among the menu options, and press the “Select” key to activate the current item.

Marks were covered in section 6.5. All other items are described below.

6.9.1 Moving to a specified time.

When a DAISY audio file is playing, to move to a specific time, press the “Select” key to open the menu, navigate to “Move By Time”, and press “Select again. The elapsed and total times are announced, and you are placed in an edit box in to which you can type the specific time you want to move to. Enter the time in 6 digits: 2 digits for the hour, 2 digits for the minute, and 2 digits for the second. Thus move to 5 minutes and 20 seconds in the current file, enter 000520. You can enter text by choosing your desired numbers with the Left and Right arrows, then pressing “Select” to enter them.

When you have finished entering the time to move to, press and hold the “Select” key, and playback resumes at the time you specified.

6.9.2 Moving by percentage.

To move to a specific position in the file according to percentage, press “Select” to open the menu, navigate to “Move By percent” and press “Select” again. The current percentage is announced, and you are placed in an edit box in to which you are prompted to type the percentage to move to. Use the usual text entry method to enter the percentage of the file you want to move to and press the “”Select” key. Playback resumes at the percentage of the file you specified.

6.9.3 Moving to a specified heading.

To move to a specific heading in the DAISY book, press “Select” to open the menu, navigate to “Move By Heading”, and press “Select” again. The current heading number and the total number of headings are announced, and you are placed in an Edit box in to which you are prompted to type the number of the heading you want to move to. Use the usual text entry method to enter the number of the heading you want to move to and press the “Select” key. The DAISY player begins playing at the position of the heading you specified.

6.9.4 Moving to a specified page.

To move to a specific page in a DAISY text book or document file, press “Select” to open the menu, navigate to “Move By Page”, and press “Select” again. The current page number and the total number of pages are announced, and you are placed in an Edit box in to which you are prompted to type the number of the page you want to move to. Use the usual text entry method to enter the number of the page you want to move to and press the “Select” key. The Book reader begins playing at the position of the page you specified.

6.9.5 Reviewing Book or File Information.

You can obtain information on the currently playing file by holding the “Explorer” key. The information displayed will depend on the type of content you are playing.

While in documents:

- File Name: announces the current file name and format.

- Page information: the current page of the current document.

- Mark information: announces the number of marks in the current file. If there are no marks, no mark information is announced.

- Percentage information: Announces the current position in the document as a percentage.

Use the Up and Down arrows to navigate among the various pieces of information. Use the Left and Right arrows to review the current piece of information character by character. To exit the “Information” dialog press the “Cancel” button.

The following information is displayed when holding the “Explorer” key when playing DAISY content:

- Title: announces the title of the current book.

- Mark information: displays only if marks are set.

- Page information: displays the current page and the total number of pages in the book.

- Heading information: announces the current heading number.

- Time info: (in DAISY audio only) displays the elapsed and remaining playback time.

6.10 Using The Explorer.

The Explorer lets you easily search for, delete, and play DAISY and document content.

6.10.1 Entering and Exiting the  Explorer.

To open the Explorer, while playing a DAISY book or  document, press the dot-like button in the center of the bottom row of keys, between the "OCR" and "Cancel" buttons. You are presented with a list of  files and folders containing DAISY and document content. To exit the Explorer, press the "Cancel" button and you are returned to the book or document you were previously playing.

6.10.2 Structure of the Explorer

By default, The Explorer sorts files and folders containing DAISY books and documents alphabetically. DAISY books are displayed first, followed by a general list of playable document folders and files. If a folder contains some files which are  supported, and some which are not, only the files of types supported by Blaze EZ are shown in the Explorer. In addition, document files contained in a folder appear under the root folder, even if they are contained in subfolders on the storage disk. For example, if you have a "My Documents" folder that contains subfolders called "Manuals" and "Recipes", as well as documents in the root, all playable items are shown under the "My Documents" folder.

If you wish to see content exactly as it is stored, you can change the “Explorer Mode” setting in the Global Settings dialog to “Directory Mode”.

6.10.3 Searching for and Playing Content.

Use the following keystrokes in the Explorer:

- Use the Up and Down arrows to move through a file or folder list.

- Use the Left arrow to move back a level in the folder structure.

- Use the Right arrow to move forward a level in the file/folder structure.

- Use the "Select" button to play the focused file or folder.

- Press and hold the Up arrow to move to the DAISY group.

- Press and hold  the Down arrow to move to the general documents group.

- Press and hold the "Cancel" button for more than 2 seconds to delete the focused file or folder. You are prompted whether you are sure you wish to delete. Press "Select" for yes, or "Cancel" for no. You can also press and hold the "Cancel" button during playback to delete the current file without entering the "Explorer".

Browsing and Playing Content:

The following describes how to search for and play content in more detail:

- When you press the "Explorer" button during playback, the current title is spoken, and you are focused on its location in the Explorer.

- To play other content in the same folder, use the Up or Down arrow to navigate to the desired content, and  press "Select" or the "Book" button  to start playback of the newly focused book or document.

- To navigate to a different folder, press the Left arrow to move out to the folder list.

- Use the Up or Down arrow to navigate to a different folder.

- Press the Right arrow to enter the folder you wish to open.

- The content of your chosen  folder is displayed. Use the Up or Down arrow to navigate the content.

- Press the "Select" button or the "Book" button to start playback of the focused item.

Moving among groups:

You may find it cumbersome and unnecessary to move through all the documents to get to your DAISY books, or to navigate all your DAISY books before getting to your documents. You can press and hold the Down arrow in the DAISY book list to quickly jump to the beginning of the documents list. When in the documents list, press and hold the Up arrow to jump to the DAISY books list.

Selecting and Acting on Files and Folders:

You can select multiple items for playback and other actions by pressing the Book Reader  key. Press and hold the Book Reader  key to cycle among additional selection options: “Select All”, “Select from current to end”, “Select from current to beginning”, and “Start continuous Selection”.

You can access a menu of file management options by pressing the Explorer key once more while the Explorer is opened.

Again. Use the Up and Down arrows to choose from “Move By” (number), “Copy”, “Cut”, and “Paste” (if items are on the clipboard). Press “Select” to activate your desired action on the selected files.

6.10.4 Continuous Playing of a Folder.

You may have a situation where you wish to play a series of files in succession. For example, if you have a text book or user manual wherein each chapter is in a separate document. You can play all the content in a given folder continuously using the steps below:

- Press the "Explorer" button and your currently playing title is displayed.

- Press the Left arrow to move to the folder list.

- Use the Up or Down arrow to navigate to the folder you wish to play.

- Press the "Select" button or the "Book" button and all content in the folder will be played in sequence.

6.10.5 Checking the path.

To check the location of the current file or folder, when focused on it in the Explorer, press and hold the "Explorer" button for more than 2 seconds. The path of the current file or folder is announced.

6.10.6 Delete file or folder.

You can easily delete a file or folder from the Explorer or during playback. When the content is playing, or when it is focused in the Explorer, press and hold the "Cancel" button. You are asked if you are sure you wish to delete the content. Press "Select" for yes, or "Cancel" for no.

7. Recording.

This section discusses how to create and listen to recordings as well as how to set your recording preferences.

You can record via the internal microphone, an external stereo microphone or line-in source, from a headset microphone, or from the FM radio. You can also adjust the quality of the recording and the sensitivity of the microphone.

7.1 Making a Recording.

To begin making a voice recording, simply press and hold the "Record" button from either the Media Player or the Home menu.

- Note: If you press and hold the "Record" button while the FM radio is playing, the unit begins recording the radio content. When the recording is started, you are notified either by a tone, a message, or not at all, depending on how you have set your Record Settings.

To pause recording, press the "Record" button quickly. Another brief press of the "Record" button resumes recording again.

To stop and save the recording, press the "Select" button. As with beginning a recording, the notification you receive depends on how your Record Settings are set. To cancel recording, press the "Cancel" button. You are asked if you wish to cancel the recording. Press "Select" to stop recording, or press "Cancel" to, in a word, cancel the cancellation, thus resuming the recording again.

The recorded file is saved in the "Record" folder of the internal flashdisk and titled with the time and date.

To record via an extermal microphone, you can simply connect it and begin recording. If you wish to use a line-in source, you should adjust the Recording Source option in the Record settings before creating a recording.

7.2 Listening to a Recorded File.

A quick press of the Record button from anywhere opens the list of recorded files created using the internal microphone or an external microphone or line-in source. Use the Up and Down arrows to navigate among the recordings, and press the "Select" button to play a recording. You can navigate recordings just as you can other audio files in the Media Player.

7.3 Searching for and Deleting Recorded Files.

You can navigate and delete recordings using the Explorer just as you can when using the Media Player and Book Reader. To access all the recordings, including those created from the radio, press the "Explorer" button while a recording is opened.

Recordings are listed in chronological order by time. Use the Up and Down arrows to move through the list of recordings. Use the Left arrow to move to the folder list, and use the Right arrow to open a folder. Press "Select" on a file you wish to play.

You can delete an individual recording by navigating to it and pressing and holding "Cancel". To Delete all recordings in the Record folder or the Radio folder, focus the folder and press and hold "Cancel".  You  are asked if you're sure  you wish to delete the  file or folder. Press "Select" to delete the file or folder, or press"Cancel" to cancel the deletion. You can also delete a recording during playback.

7.4 Setting Recording Options.

You can change the Record settings to adjust microphone sensitivity, recording quality, recording source and the location where recordings are stored.

To open the "Record Settings" dialog, from the Home menu, press the Left or Right arrow to navigate to the Utilities menu, and press “Select”. Use the Left and Right arrows again to navigate to "Record Settings" and press "Select". You are placed on the first item, "Recording quality". Use the Up and Down arrows to move among the settings in the list, and use the Left and Right arrows to change the value of a setting.

The available settings and values are as follows:

1) Recording Quality: High Quality Wav, High Quality MP3, Standard quality, Speech quality and User Defined.

- High Quality WAV: sample rate = 44100Hz, bit-level = 16Bit, channels = stereo

- High quality mp3: sample rate = 44100Hz, bitrate = 320kbps, channels = stereo

- MP3 normal mode: sample rate = 44100Hz, bit rate = 192kbps, channel =  stereo

- Speech quality: sample rate = 22050Hz, bitrate = 64kbps, channel =  mono

- User Defined: Press the Down arrow to display additional options for the recording format and sample rate, bit rate, and specify them according to your wish.

2) Record Method: Microphone, Headset microphone, and Line-in.

- Microphone: built-in and external microphones: automatically detects the recording source.

- Headset Microphones: Apple compatible headphones including  built-in microphones.

- Line-in: refers to using a line-in cable to connect your CD player, cassette player, or other source to directly record the audio output.

- Note: If Line-in or Headset Microphone is chosen, and the device is not connected, recording is automatically done using the internal microphone.

3) Microphone Sensitivity: levels 1-5. Use level 1 to record louder sounds such as a performance in a concert hall, and use higher sensitivity levels in quieter environments such as when  recording a class lecture from the back of the room.

4) Record notification: Beep, Message, or Off. This setting determines what types of notifications you receive when beginning, pausing, and ending recordings.

5) Recording Save Disk: Flashdisk or SD Card. This setting determines the location in which recordings are stored.

8. OCR.

"OCR" refers to Optical Character Recognition, which simply means the recognition of letters in an image, and the conversion of these letters to text. The Blaze EZ accomplishes this by snapping a photo of a page of text with it's built-in 5 mega-pixel camera, and processing it through its included OCR software.

8.1 Opening and Closing OCR.

To open the OCR program, press the “OCR” button on the bottom right of the Blaze EZ's keypad. It is labeled with an “o” in Braille. The OCR menu is displayed, and you are placed on the first item, "Take a picture". Use the Up or Down arrow to navigate among the items in the OCR menu.

The OCR menu contains the following items:

- Take a Picture:

- Flash On/Off: use the Left and Right arrows to adjust the setting.

- Results: opens a list of recognition files from previously recognized images.

- Open a taken file: Allows you to open a previously-taken photo and have the text re-recognized.

- Open an External File: lets you open and recognize other image files not captured on Blaze EZ.

- Options: Used to set how images are processed and files are saved.

To Close the OCR program, press the "Cancel" button and you are returned to the Home menu.

8.2 Taking a Picture.

You can capture an image using the Blaze EZ camera in 2 ways: either by physically holding the Blaze EZ in an optimal position for taking a picture, or using the OCR stand included in the optional Premium Accessory Pack.

Taking a picture without the OCR stand:

- Place your elbows on either side of the printed material.

- Hold the Blaze EZ in your hands in the horizontal position where your hands meet in the center of the arch made by your elbows.

- Press the "OCR" button to open the OCR program.

- Press the "OCR" button once again to take the picture.

- Blaze EZ auto focuses, shoots the picture, and automatically begins text analysis. During text analysis, progress beeps are heard, and when the analysis is finished, Blaze EZ automatically begins reading the text aloud.

- Note: To take a decent picture of a normal sheet of paper, the camera should be centered about 9 to 12 inches above the page. It should be placed lower for smaller items.

Using the OCR stand:

- Place the stand in the horizontal position, so that it's wall faces to the right.

- Place the Blaze EZ in the tray on the top of the stand so that the camera lines up with the small hole in the bottom.

- Line up the printed material so that it rests against the edge of the inside wall of the OCR stand.

- Press the OCR button to take the picture.

8.3view Guidance.

Blaze EZ offers several options to assist you in taking accurate scans, specifically when not using the OCR stand. These settings are located in the “Options’ dialog of the ‘OCR” menu. When the OCR program is opened, use the Up or Down arrow to navigate to ‘Options’ and press “”Select”. Use the Up and Down arrows to move among the options, and use the Left and Right arrows to change the value of a setting.

Most of the options in this dialog will be discussed in section 8.7. However, we will discuss here the various options related to determining camera placement as it relates to the text.

- "Preview Area" (provides direction for better focusing the camera. When this option is on, as you hold the EZ above the text, you will receive directions like “Move up and right”, or “Move down”, etc.

- "Image area" (used when text color does not contrast with background, or when scanning items like brochures where text is mixed with photos and does not contain standard lines or columns).

- "View orientation" (power button to the right, or power button to the left)

- "Focus detection notification method” (determines how you are alerted that the text is in focus: Message, Beep, or All)

When you have chosen your settings, press “”Select” to save them. Press “Cancel” to return to the “OCR” menu without saving your settings.

8.4 Taking continuous pictures.

You can take multiple pictures continuously and save them in a single file. You may wish to do this if you are scanning a book or document with multiple pages which you wish to scan quickly and continuously and save in a single file.

You must first enable the “continuous scanning” option in the “Options” dialog of the OCR menu. When you open the “Options’ dialog, ‘continuous scanning” is the first option. By default, it is off. Use the Left and Right arrows to choose among “off”, “Manual” and 1 second increments from 5 seconds to 9 seconds.

When this option is off, pictures are always captured and saved individually. If it is set to “manual”, images are captured when you press the OCR button, but scans are saved in a continuous file until you press the “Select” button to analyze and save. If you set “Continuous scanning” to one of the time elements, Blaze EZ automatically takes a shot at each of the specified time intervals, and saves the scans to a single file until the “Select” key is pressed.

Once you have set your “Continuous scanning” preference, arrow down to “How to Save” and choose the format of the file name under which OCR scans are saved. “date” saves each file according to the date and time it is created. “Custom” allows you to specify a file name under which to save the scan. If you choose “Custom” arrow down once more and enter your desired file name in to the resulting edit box.

Press “”Select” to save your settings and you are returned to the OCR menu. If you have chosen to enable continuous scanning options, the “Take a Picture” item now reads “Take continuous pictures every --- seconds”. Press the “OCR” key to begin capturing images.

You can pause continuous scanning by pressing the OCR button once more. When you are finished capturing continuous images, press the “Select” button to analyze and save the images.

Press “Cancel” to cancel the continuous scan.

8.5 Reading the OCR Result.

You can access OCR results in several ways:

As described above, after an image is captured, Blaze EZ automatically analyzes and reads the text aloud.

You can also access recognition result files via the "Results" option in the OCR menu. To open a result file via the menus, do the following:

- Press the "OCR" button to open the OCR program. The "Take a picture" option is displayed.

- Press the Down arrow to navigate to "Results", and press "Select".

- The list of recognized files is displayed according to the date and time they were created.

- Press the Left or Right arrow to select the file you wish to read, and press "Select" to play it.

While playing a recognized file, you can also access the results by pressing the "Explorer" button. Result files are displayed in descending order. Use the Up and Down arrows to navigate the list, and press "Select" on a file you wish to play.

Press the OCR button while in the Explorer to select multiple files.

Hold OCR to cycle other selction options: “Select All”, “Select from current to end”, “Select from current to beginning”, and “Start Continuous Selection”.

While reading a recognized file, press the "Select" button to pause and resume reading. You can navigate the file in detail in the same way as you would a document file in the Book Reader.

Pressing the "OCR" button takes a new picture. Press the "Cancel" button to return to the OCR menu. Press and hold the "Cancel" button to delete the currently playing file. You are asked if you are sure  you want to delete. Press "Select" to delete, or "Cancel" to cancel deletion.

8.6 Importing Image Files.

You can open image files previously taken using Blaze EZ, or you can open image files from other sources and recognize the text.

8.6.1 Opening a Captured File.

To open a file captured on the Blaze EZ, follow these steps:

- Open the OCR menu.

- Use Up or Down arrow to navigate to "Open a taken file" and press "Select".

- The list of JPG files taken on the Blaze EZ will appear titled using the date and time they were captured.

- Use the Up or Down arrow to navigate to the file you want to re-recognize, and press "Select".

- The file is analyzed, during which time you will hear beep sounds to indicate progress.

- When the recognition is complete, the text is automatically read aloud.

8.6.2 Opening an External Image File.

As mentioned previously, you can open image files created from other sources, and recognize any available text in these images using Blaze EZ. Blaze EZ currently recognizes text from JPG files and text-based PDF files.

To open and recognize an external image file, follow these steps:

- Open the OCR menu.

- Press the Up or Down arrow to navigate to the "Open An External File" option.

- Blaze EZ displays a list of image files.

- Use the Up or Down arrow to navigate to the file you want to recognize and press "Select".

- The file is analyzed, during which time you will hear beep sounds to indicate progress.

- When the recognition is complete, the text is automatically read aloud.

8.7 Setting Other OCR Options.

8.7.1 Turning Flash On/Off.

You may need to turn the flash on or off depending on the brightness of your surroundings. If recognition seems to be poor, try turning the flash off to establish more appropriate lighting and better recognition.

To change the flash setting, do the following:

- Open the OCR menu.

- Use the Up or Down arrow to navigate to "Flash On/Off".

- Use the Left or Right arrow to change the setting.

8.7.2 Selecting Image Recognition File handling.

Use this option to choose what happens when files are recognized.

- Open the OCR menu.

- Press the Up arrow to navigate to "Options" and press "Select"

- Use the left or Right arrow to choose to save or delete files as they are captured and recognized.

- Press "Select" to save your settings.

- To cancel saving your settings, press "Cancel" and you are returned to the OCR menu.

8.7.3 Deleting All Recognized Text Files.

You can use the "Delete all Files" option to delete all recognized text files since executing the OCR program. This may be useful if you've experimented recognizing various things that you do not wish to keep, but you want files you saved before to remain.

- Open the OCR menu.

- Use the Up or Down arrow to navigate to "Options" and press "Select".

- Use the Up or Down arrow to navigate to "Delete All Files" and press "Select".

- When asked if you want to delete all files, press "Select" to delete the files, or press "Cancel" to cancel the deletion.

- Note: This option deletes all files captured in the current session, not all files captured by the OCR program.

9. Web Radio.

Web Radio is live audio casting over the Internet as opposed to an FM signal. Web streaming is the method by which devices like Blaze EZ allow you to listen to such casts. To use the Web Radio, Blaze EZ must be connected to a wireless network.

9.1 Executing and exiting Web Radio.

To execute the Web Radio application, from the Home menu, use the Left or Right arrow to navigate to Web Radio” and press “Select”. When the Web Radio application is launched, Blaze EZ searches for Web Radio station playlist addresses stored on the unit and creates a channel list. When the channel list  creation is complete, you are placed on the channel you last listened to. If you have not previously listened to the Web Radio, you are placed on the first station in the list. Use the Left and Right arrows to navigate among the channels, and press “Select” on a station  you wish to play.

For your convenience and enjoyment, Blaze EZ comes preloaded with several Web radio stations local to your region or language. However, you may add your own favorite web radio stations and/or delete those currently loaded on the unit.

During playback, press “Cancel” to return to the channel list. Press “Cancel” from the channel list to close the Web Radio and return to the Home menu.

If the Web Radio does not play, please check the following:

- Make sure you are connected to the Internet.

- Be sure the Webradio folder on Blaze EZ contains proper Web Radio channel playlist addresses. Blaze EZ supports m3u and PLS radio playlists.

- The URL may no longer be valid, or the server may be experiencing temporary problems. Unfortunately, Web Radio stations often change their Url’s thus, it is often necessary to check and change the address you are using to connect  to a Web Radio channel.

9.2 Moving by channel.

During Playback, you can use the Left and Right arrows to move among your Web Radio channels. As you use the arrows to navigate, Blaze EZ stops playing the channel you are currently listening to, and connects to the channel to which you have just navigated. In other words, when you navigate while streaming, you are, in escence, live tuning, similar to turning a dial on an FM radio. As explained above, you can also press “Cancel” and navigate the channel list, and press “Select” on the station you actually wish to listen to. You may wish to use the second method if you want to skip through many stations quickly.

Because Web casting happens in real time, you are not able to move by track or other time elements as you can when playing stored audio files. The movement units available when playing Web Radio channels allow you to move by1 channel, 5 channels or to the first or last channel. Use the Up and Down arrows to choose the movement unit, and use the Left and Right arrows to move by the chosen unit of navigation.

9.3 Searching for Web Radio Channels.

You can search for stations from a database of Web radio channels by pressing the “Explorer” button. However, before you can search this database, you must first download it to the unit. To download the most up-to-date database, press and hold the “Explorer” button. When the database is loaded, you are returned to the channel list, from which you can press the “Explorer” button quickly to begin searching the database.

You are presented with 2 search modes: “Category” and “Word Input” mode. Use the Left and Right arrows to move between them.

- Searching by Word:

To search by word, use the Left or Right arrow to move to “Word Input Mode”, and press the Down arrow to access the “Input Search keyword” edit box. Press “Select” to start inputting text.

In the edit box, use the Up or Down arrows to move among the available keyboards: Upper Case Letters, Lower Case Letters, Numbers, and Symbols. Use the Left and Right arrows to navigate the characters in a keyboard, and press "Select" on the character you wish to enter. Press the "Record" button to delete a character, and press the "Explorer" button to read what you have typed into the edit box. When you have finished entering thesearch text, press and hold the "Select" button for several seconds. You are returned to the “Input Search Keyword” button and your text is spoken. Press the Down arrow to search and display the results for the text you typed.

- Searching by category:

To Search the Web Radio database by category, use the Left or Right arrow to navigate to “Category Mode”. Press the Down arrow to get a list of countries from which radio stations originate.

Use the Left and right arrows to move among the countries. Press the Down arrow to get a list of languages in which Web radio stations are available in a given country. Use the Left and Right arrows to move among the languages. Press the Down arrow again to get a list of genres available in the selected language.

Once again, use the left and right arrows to move among the genres, and press the Down arrow to get a list of stations in the selected genre. You can press the Up arrow to move back to genre, language or country and use the left or Right arrows to choose a different category.

In the list of channels available according to your sorting choices, press the left and Right arrows to move among the results. If you want to add an item to your channel list, press “Select” on it. Blaze EZ announces “Selected” before the item.

You may select multiple channels in a results list. Once you’ve selected all the channels you want to add, hold the “Select” button for more than 2 seconds. Blaze EZ announces, “Creating channel list”, soon followed by “Channel list creation complete”. You are placed in your channel list again, and the selected items are added.

10. Podcasts.

Podcasts are audio files published by individuals or organizations via a method called RSS. Podcasts are available on any number of topics including news, politics, music, books, technology, cooking, and an unlimited number of individual casts covering very specific personal interests. Anyone with the know-how and the right equipment can publish a podcast, thus, the selections are virtually unlimited.

RSS, is short for “Really Simple Syndication” and allows individuals to receive updated web content via “feeds” to which people can subscribe through applications called RSS Readers or Pod Catchers which access the feed addresses and bring down the available content as it is updated. Users can then see the available podcast content and choose to download the podcast files if they wish to listen to them. The Podcasts application on the Blaze EZ  is such a pod catcher.

10.1 Subscribing to Podcasts.

Podcast feed address information must be stored in the “Podcast” folder on the Blaze EZ internal flashdisk. You can paste the Podcast XML addresses directly, or copy an OPML created by another podcatching application, and the podcasts will be imported automatically when you next open the Podcast application on the Blaze EZ.

If you paste podcast feed addresses directly, each URL must be placed on it’s own line in a file called “podcast.url” for the Blaze EZ to properly recognize it.

So that you can easily explore the potential of the “Podcasts” application and the variety of content available, several podcast feeds are pre-loaded on Blaze EZ when it ships. However, you may add your own favorite feeds, or delete those that do not appeal to you.

You can also search for and subscribe to podcasts directly on the Blaze. When the Podcasts application is opened, press the Explorer key to open the search dialog.

You are placed in a combo box in which you can choose to search either by category, or “Word input mode”, in which you can type a search term. Use the Left and Right arrows to choose the mode, and press the down arrow.

If you choose “Word input mode”, type your search term and press the Down arrow again to perform the search. If you choose “Category Mode” use the Left and Right arrows to choose your category, and press the Down Arrow to initiate the search.

When the search is complete, you are placed in a list of results. Use the Left and Right arrows to navigate among the podcasts that matched your search. Press and hold “Select” to add the current feed to your feed list. To select multiple items, press “Select” briefly on each of the items you want to select, and press and hold “Select” to add all the selected items to your feed list.

10.2 Executing and Exiting Podcasts.

To open the Podcasts application, from the Home menu, use the Left or Right arrow to navigate to "Podcasts", and press "Select". Blaze EZ announces, "Creating Feed list" and progress beeps are heard while the unit creates the list of feeds from XML addresses pasted into the Podcasts folder, and/or from OPML files placed there. If you are not connected to the Internet, you are prompted and asked if you wish to set up Wi-fi. Press "Select" to open the "Network Setup" dialog described previously, or press "Cancel" to open the Podcasts application without establishing a Wi-fi connection.

The first time you open Podcasts after pasting an OPML file into the Podcasts folder, it will take more time to create the feed list. When Blaze EZ is finished creating the feed list, it announces "Feed list creation complete" and you are placed in the list of podcast feeds. To close the Podcasts application, press the "Cancel" button, and you are returned to the Home menu.

10.3 Navigating Feeds and Podcasts.

From the podcast feed list, use the Left and Right arrows to navigate among the podcast feeds. To view the content for a feed, press the Down arrow, or the Select key.

If you are connected to the Internet, the feed is updated. Progress beeps are heard to let you know the update is in progress. When the update is complete, you are placed in a list of podcasts available for the feed. If you are not connected to the Internet, when you press the Down arrow or "Select" button on a feed, you are given the list of podcasts available the last time you updated the feed.

Use the Left and Right arrows to navigate among the podcasts in a list. As you navigate, the  title of the podcast is announced as well as its status, "Not downloaded", "Not Played", "Partially played" or played.

If the podcast has not been downloaded, press "Select" on it to begin downloading.  (You must have an active Internet connection to download podcasts.)  Progress beeps are heard, and progress percentages are announced as the podcast downloads. When the download is complete, the unit begins playing the podcast in the Media Player.

If the podcast has been downloaded, if you press "Select" on it, it will begin playing in the Media player. If you have already listened to part of the podcast, it will resume playback at the position at which you last stopped playing it. While playing a podcast, press "Cancel" to stop playback, and return to the podcast list.

10.4 Deleting Content.

You may delete a podcast feed, or an individual podcast by navigating to it and pressing and holding the "Cancel" button for more than 2 seconds. You are asked if you're sure you want to delete the item. Press "Select" to delete, or press "Cancel" to cancel the deletion.

11. Library Services.

The “Library services” menu allows patrons of libraries that use the online DAISY protocol or other online interfaces to access and download content directly to the Blaze EZ. The presence of this menu, and the options in it depend on the country in which you are located and the libraries supported. We will continue to add librarie services as we become aware of their existence.

Note: in regions where there is no supported Online library for the blind, this menu does not appear on Blaze EZ. If you wish to recommend an Online library for inclusion in our support, please contact us at hims@himsintl.com, or support@hims-inc.com.

11.1 Logging In.

Every program in the “Library services” menu opens at the login screen. To log into your library account, please do the following:

- From the Home menu, use the Left or Right arrow to navigate toLibrary Services, and press "Select".

- Use the Up and Down arrows to navigate to your desired library, and press “Select”.

- You are placed in the Login dialog in the ID edit box. Use the Up or Down arrow to move among the items in the dialog. Press "Select" on an item to bring up the keyboards and enter text.

- In the edit box, use the Up or Down arrows to move among the available keyboards: Upper Case Letters, Lower Case Letters, Numbers, and Symbols. Use the Left and Right arrows to navigate the characters in a keyboard, and press "Select" on the character you wish to enter. Press the "Record" button to delete a character, and press the "Explorer" button to read what you have typed into the edit box. When you have finished entering text, press and hold the "Select" button for several seconds.

- Once you have entered your ID and password in the manner described above, use the Down arrow to navigate through the other fields.

- Choose whether or not to save both your ID and Password. Use the Left or Right arrow to navigate to Yes" or "no".

- Depending on your library, there may be other options for which you must make choices via the left and Right arrows.

- Arrow down to the "Login" item, and press "Select" to log in.

11.2 Navigating content.

When you are successfully logged into your account, you can search for the content available in various categories. The available categories will depend on the library you are viewing, however, no matter what library service you are using, you  are placed in the category list after logging in, and you can move among the categories using the Left and Right arrows.

Once you have selected a category using the left or Right arrow, press the Down arrow to access sub categories if they are available, or the list of books in your chosen category. Pressing the Down arrow always takes you to a more specific designation, while pressing the Up arrow returns you to a more general designation. Use the Left and Right arrows to navigate the items in the sub category or book list.

The last item in the dialog lets you choose the storage location or download path of books downloaded from the current library. To change the download path, press the  Down arrow to ‘Storage location’ or “Download Path”, and use the Left and Right arrows to choose between “Internal Disk’ and “SD Card”.

11.3 Downloading and Playing content.

If you press “Select” on a particular book, you are presented with 2 items: “Download Book” and “Book information”. Use the Left and Right arrows to move between the two options.

The Book information” list provides information about the book such as: ‘Title”, “Publisher”, “Creator”, “Date”, “Description” and “Size”. Use the Up and Down arrows to move among the items in the information list. Use the left and Right arrows to read an item in the information list character by character. Press the “Cancel” button to exit the information dialog and return to the book list.

To download a book, navigate to it using the left and Right arrows, and press “Select”. Then press “Select” on “Download Book”. While the book is downloaded, Blaze EZ announces progress in the form of a percentage.

When the download completes, Blaze EZ gives you the option of opening the book. The Book reader application is launched and the book begins playing. If you press “Cancel”, you are returned to the book list you were viewing when you initiated the download.

You can access books downloaded using the Online DAISY program at any time using the Book reader’s Explorer.

11.4 Returning content to the Online DAISY Bookshelf.

If your library uses the online DAISY protocol, you are only allowed to have a certain number books on loan at a given time, and required to return books to the library before you can borrow more of them. To return content to the virtual library, simply locate it in the book list of the Online DAISY program, and press and hold “Cancel” to return the book. The book is deleted from the unit and returned to the library.

12. Utilities.

The Utilities menu is the next item in the Home menu. To open it, Use the Left or Right arrow to navigate to "Utilities" and press "Select".

The "Utilities" menu contains the following items:

- Color Reader

- Alarm

- Manual Scan disk.

- Play Audio CD" (Only appears if CD-ROM is connected.)

- Upgrade.

- Initialize Options.

12.1Color Reader.

The Color reader app allows you to capture an image under the camera and receive a report of the color or colors within it as well as their RBG values.

12.1.1 opening the color reader.

From the Home menu, use the Left or right arrow to navigate to ‘Color Reader” and press “Select”.

12.1.2 Using the color reader.

When the Color Reader is opened, the unit announces “Ready” and places you on “Capture Image”. Press the “Select” key to capture the image under the camera. After a few seconds, the unit announces the color or colors in the image. You can arrow down to have the RGB values announced.  You can also use the Up and Down arrows to navigate to “Capture Image” and press “Select” to find out the color of a different item.

Be aware that lighting greatly impacts the accuracy of the color result. If you have low light, you should turn flash on in order to have enough light to ascertain the color accurately. If you have a well-lit environment, you should turn the flash off in order not to flood the image with too much light. To turn the flash on or off, use the up or Down arrow to navigate to “Flash” and use the Left or right arrow to switch flash on or off.

When you have finished using the Color Reader, press “Cancel” to exit and return to the “Home” menu.

12.2 The Alarm.

Use the Blaze EZ’s alarm to wake up in the morning or remind you of an event.

12.2.1 Setting an alarm.

From the Utilities menu, use the Left and right arrows to navigate to ‘Alarm” and press “Select”. You are placed on “Alarm Settings’. Press “Select” again to open it.

The “Alarm Settings’ dialog appears and you are placed in the ‘Hour” edit box.Use the left and Right arrows to choose your hour, and arrow down to the “Minute” edit box. Use the left and Right arrows to choose the minute, and arrow down. Use the Left and Right arrows to choose between AM and PM and arrow down once more.

The remaining items in the dialog are various options you can set in regards to your alarm. Use the Up and Down arrows to navigate among the settings, and use the Left and Right arrows to adjust the value of each setting.

The “term” option allows you to choose how often the alarm will sound; Once, each weekday, or on the weekend.

The Alarm Sound lets you choose among the available alarm sounds on the Blaze ET.

The Alarm volume can be set using values 1 to 15.

The Alarm interval determines the amount of time between repetitions of the alarm sounding. Choose among 3, 5, 10, 15, and 30 minute intervals.

Finally, the “Term Number” setting lets you choose the number of times the alarm will go off if you do not cancel it. Choose among 1, 3, 5 and 10 times.

When you finish selecting your options, press the “Select" button to save the alarm.

12.2.2 Deleting an alarm.

If an alarm has sounded at its specified time, there are no more recurrences, and you have canceled the alarm, the alarm is deleted. However, if you want to delete an alarm that is expected to sound at a later time, you can do so using the “delete Alarm” option.

When you open “Alarm” from the Utilities menu, you are placed on “Alarm Settings’.

Arrow down to “Delete Alarm” and press “Select”. The alarm is deleted.

12.3 Format.

The format option allows you to clear any disk connected to the Blaze of all its data and cleanly format it.

To format the disk, follow these steps:

- From the Home menu, press the Left arrow to navigate to "Utilities" and press "Select".

- Use the Left and Right arrows to navigate to “Format” ", and press "Select".

- The unit announces, "Format  disk", and places you in a combo box from which you can choose which disk to format.

- Use the Left and Right arrows to choose the disk, and press “Select”.

- You are prompted that this will erase all data. Press “Select” to continue.

- The disk will be formatted, and you are returned to the Utilities menu.

12.4 Playing an Audio CD.

You can play an audio CD using Blaze EZ by connecting an external Cd-rom drive to the USB OTG port of the Blaze EZ using the included USB gender adapter. 
Use the following steps to play an audio CD on Blaze EZ:

- Connect the CD-ROM drive to the USB OTG port of Blaze EZ.
2)from the Home menu, press the Left or Right arrow to navigate to “Utilities” and press “Select”..

- Use the Up or Down arrow to navigate to “Play Audio CD” and press “Select”. (Option only appears if CD-rom drive is connected.

- Use the Up and Down arrows to navigate the CD content and press "Select" on your desired track to begin playback.

- Use normal playback and movement commands to move through the content on the CD.

12.5 Upgrading Blaze EZ firmware.

This section describes how to upgrade the firmware on the Blaze EZ. You may upgrade the firmware in 2 ways: Online, or Offline.

12.5.1 Online Upgrade.

Use the following procedure to upgrade your Blaze EZ online.

- Make sure you are connected to the Internet. See chapter 10 for details on setting up a wireless network connection.

- Connect the AC adapter. If you try to upgrade without connecting the AC adapter, the Blaze EZ will notify you that the AC adapter is not connected, and will cancel the upgrade process.

- From the Home Menu, use the Left or Right arrow to navigate to “Utilities” and press “Select”.

- Use the Up or Down arrow to navigate to "Upgrade" and press "Select".

- You are prompted to choose how you wish to upgrade. Use the Left or Right arrow to navigate to Online and press "Select".

- If you are connected to the Internet, and a mew upgrade is available, the download begins. If you are not connected to the Internet, you are notified and taken to the "Network Setup" dialog. If the current upgrade is the latest, you are notified, and asked if you wish to proceed. Press "Select" to proceed, or press "Cancel" to cancel the upgrade process.

- When the download is complete, the unit announces, "Start System update", and the update begins installing. During the download, you may press the "Cancel" key to cancel the download process, and you are returned to the upgrade options list. However, once the upgrade begins installing, please do not press any keys on the Blaze EZ or disconnect it from the AC adapter.

- When the installation is complete, Blaze EZ announces that the upgrade was successful, and reboots to the Home menu.

12.5.2 Offline Upgrade.

Use the following steps to upgrade your Blaze EZ offline:

- Connect the AC adapter. If you try to upgrade without connecting the AC adapter, the Blaze EZ will notify you that the AC adapter is not connected, and will cancel the upgrade process.

- From the Home Menu, use the Left or Right arrow to navigate to “Utilities” and press “Select”.

- Use the Up or Down arrow to navigate to "Upgrade" and press "Select".

- You are prompted to choose how you wish to upgrade. Use the Left or Right arrow to navigate to Offline and press "Select".

- The unit announces, "Start System update", and the update begins installing. Once the upgrade begins installing, please do not press any keys on the Blaze EZ or disconnect it from the AC adapter.

- When the installation is complete, Blaze EZ announces that the upgrade was successful, and reboots to the Home menu.

12.6 Initialize Options.

The “initialize options” item lets you restore many of your Blaze EZ settings back to the factory defaults.

To initialize the options, follow these steps;

- From the home menu, press the left or Right arrow to get to “Utilities” and press ‘Select”.

- Use the Up or Down arrow to get to ‘Initialize options” and press “Select’.

- Blaze EZ prompts, “initialize options/ Yes.” Use the Left and Right arrows to navigate between ‘Yes” and “No”.

- Press “Select” on “Yes” to initialize your options. Blaze prompts, “Starting initialization, the device will reboot.”

- The device reboots, places you at the home menu, and the options are restored to their defaults.

Alternatively, you can initiate this process by holding the “OCR” button while pressing reset. When the unit beeps, release the “Reset” button, and continue to hold OCR for 5 seconds. The unit reboots, initializes the options, and reboots once more.

13. The options menu.

The “Options” menu lets you set preferences on the Blaze EZ.

To access these options, use the left or right arrow to navigate to “Options’ in the “Home” menu and press “Select”. Use the Left and Right arrows to move among the menu items, and press “Select” to open one of them. The “Options’ menu contains the following items:

- Global Settings

- Guide Voice Settings

- Sound Settings

- Record Settings

- Bluetooth Settings

- Set Wireless LAN

The Gide Voice Settings are covered in the basic functions chapter of this manual. Record Settings are covered in the chapter on recording. The remaining menu items are detailed in the following sections.

13.1 Global Settings.

The “global Settings’ dialog contains several general settings for customizing the operation and output of blaze EZ. Open the “Options” menu from the “Home” menu. Press “Select” on the first item, “global Settings’.

In the “Global Settings’ dialog, use the Up and down arrows to navigate among the various settings. Use the Left and Right arrows to change the value of a setting.

The “global Settings” dialog contains the following items:

- Mark alert: determines whether Blaze EZ announces when marks are present as you play books, documents and media. Choose On or Off.

- Number of list items per page movement: determines how many list items the Move By Page function moves by in playlists, explorer lists, etc. Choose among 5, 10, 15, 20 or 25. The default is 25.

- Power Saving: determines after what period of inactivity the unit automatically powers off. Choose among 5 minutes, 10 minutes, 15 minutes, and 30 minutes and Off. If you choose “Off”, the unit does not shut down, even after long periods of inactivity, unless the battery is low.

- Date/Time Output: Choose how the date and time are announced. Choose to announce just the date or time or to announce both.

- Language: Lets you choose the language of the Guide Voice from among several English dialects including US, UK, Australian and Indian English. Note: you must install the Australian and/or Indian English TTS data for these languages to be available.

- Book Voice: use this option to set the language of the book reading voice. The options are the same as those for the guide voice.

- Book Voice Gender: Use this option to choose whether the Book Voice uses male or female. (only available for languages that offer both male and female voices)

- Explorer Mode: Choose to view only the content playable by the current program (Content Mode), or to view all files and folders (Directory Mode).

When you have finished setting the options, press the “Select” key to save your settings. Press the “Cancel” key at any time to exit the “Global Settings’ dialog without making changes.

13.2 Sound Settings.

The “Sound Settings’ dialog allows you to set how the Blaze ET alerts you of various things such as progress, whether it notifies you at the top of the hour, and battery status as well as what sound it makes when you press buttons. Open the “Options” menu, arrow right to “Sound Settings” and press “Select” to open the dialog.

Use the Up and Down arrows to move among the various settings. Use the Left and right arrows to change the value of the current setting.

When you have finished adjusting your settings, press the “Select” key to save your changes. To exit the dialog without making changes, press the “Cancel” key.

The “Sound Settings’ dialog contains the following items;

- Audio alert settings: choose between On and Off.

- Power On/Off sound settings: Choose among beep, message, and Off.

- Button Sound settings: choose among Basic sounds 1, 2, and 3 or Off.

- Battery Sound Settings: choose between Beep and Message.

- Hourly Announcement: choose among Beep, Message and Off.

- Home Menu Alert: Choose between on and off.

13.3 Using a Bluetooth Audio Device.

You can use Blaze EZ with BLUETOOTH headsets or speakers for high quality, wireless audio listening.

- Note: If Wi-fi and Bluetooth are running at the same time, the unit’s performance may be sluggish.

13.3.1 Turning Bluetooth On/Off.

To turn Bluetooth on or off, do the following:

- From the Home menu, press the Left or Right arrow to navigate to the Utilities menu and press ‘select”. Press the Left or Right arrow to navigate to Bluetooth Information item.

- press the "Select" button to open the "Bluetooth Settings" dialog box.

- You are placed on the Bluetooth status setting. Use the Left or Right arrows to move between On and Off.

- Press "Select" to save the settings.

- Press the "Cancel" button to cancel saving the settings and return to the Utilities menu.

13.3.2Connecting a Bluetooth headset or Speaker.

Before you can connect to a Bluetooth headset or speaker, you must first locate the Bluetooth device. Use the following steps to search for a Bluetooth audio device:

- Make sure your Bluetooth audio device is in discovery mode. How to do this will differ between make and model. Please consult your headset or speaker's documentation for information about how to place your device in discovery mode.

- From the Home menu, press the Left or Right arrows to navigate to the Utilities menu and press “Select”. Use the Left or Right arrow to navigate to the Bluetooth Information item.

- Press the "Select" button to open the "Bluetooth Settings" dialog box.

- If Bluetooth is off, press the Left or Right arrow to navigate to "On".

- Press the Down arrow to initiate the search for Bluetooth audio devices. Progress beeps are sounded to let you know the search is in progress.

- When the search is complete, a list of available Bluetooth audio devices appears. If there is more than 1 available device, use the Left and Right arrows to navigate to the device you want to connect to.

- Press the "Select" button on the device you want to connect to. If the connection is successful, a series of musical tones are heard, and sound should begin outputting to the Bluetooth headset or speaker.

- Press the Cancel button to cancel connecting to a Bluetooth device and you are returned to the Utilities menu.

13.3.3Disconnecting a Bluetooth Headset or Speaker.

You may wish to disconnect from one Bluetooth device so that you can connect to another. To disconnect from a Bluetooth device, use the following steps:

- From the Utilities menu, press the Left or Right arrow to navigate to the Bluetooth Information item.

- Press "Select" to open the "Bluetooth Settings" dialog box.

- Press the Down arrow to initiate the search for devices. Progress beeps are sounded to let you know the search is in progress.

- When the search is completed, the available devices  are displayed. Use the Left or Right arrow to navigate to the currently connected device.

- Press and hold the "Cancel" button for 5 seconds. The device is disconnected and the Bluetooth profile is deleted.

- Press the "Cancel" button to close the "Bluetooth Settings" dialog box and return to the Utilities menu.

13.4 Using Wireless LAN.

This section describes how to check Wireless status, turn Wi-fi on and off, and how to connect to and delete profiles.

Using Wi-fi enhances the abilities of Blaze EZ in the following ways:

- Stay synchronized with a time server, always ensuring the Date and Time is as accurate as it can be.

- Access Web radio stations from around the globe:

- Subscribe to and download podcasts.

- Easily upgrade your Blaze EZ software online.

- Access and download books from Online DAISY libraries.

- Note: If Wi-fi and Bluetooth are running at the same time, the unit’s performance may be sluggish.

13.4.1 Checking Wireless status.

You can easily check the status of Wireless LAN simply by navigating to the Wireless Lan information in the Utilities menu. Use the Left or Right arrow to navigate to Wireless LAN information, and the Blaze EZ announces whether wireless is on or off, and, if on, if it is connected to a network.

13.6.2 Turning Wireless On/Off.

You must have Wireless LAN turned on and connected to a network to access online library Services, to listen to Web Radio, to download podcasts, and to upgrade Blaze EZ using the online update server. To turn on Wireless LAN, do the following:

- From the Utilities menu, use the Left or Right arrow to navigate to Wireless LAN information, and press "Select".

- You are placed in the "Network Setup" dialog box on the Wireless status item. Use the Left and Right arrows to change the Wireless status between On and Off.

- Press "Select" to save the setting.

If you have  previously connected to a network and it is in range, Blaze EZ will automatically detect and connect to it once wireless is turned on. You may wish to keep Wi-fi turned off when you are not using it, as this will save battery life.

13.4.3 Connecting to a Wireless network.

Use the following steps to connect to a Wireless network:

- From the Utilities menu, use the Left or Right arrow to navigate to Wireless LAN information, and press "Select".

- The "Network Setup" dialog box is opened.

- If Wireless is not turned on, use the Left Or Right arrow to change wireless status to On.

- Press the Down arrow.

- The list of available wireless networks is displayed.

- Use the Left and Right arrows to navigate among the networks. As you move through the list, the network name, signal strength, and security are announced.

- Press "Select" on the network you wish to connect to. If no password is required, Blaze EZ begins trying to connect to the network. If a network key is required, the "Password" edit box appears. In the edit box, use the Up or Down arrows to move among the available keyboards: Upper Case Letters, Lower Case Letters, Numbers, and Symbols. Use the Left and Right arrows to navigate the characters in a keyboard, and press "Select" on the character you wish to enter. Press the "Record" button to delete a character, and press the "Explorer" button to read what you have typed into the edit box. When you have finished entering the password, press and hold the "Select" button for several seconds. Press "Cancel" to cancel setting up the connection.

- After a few seconds, if the connection is successful, Blaze EZ announces that it is connected to the network and you are returned to the Home menu. If the Blaze EZ is unable to connect to the network, it announces "Connection failed", and you are returned to the list of available wireless networks.

13.4.4 Deleting a Wireless Profile.

You may find it necessary to delete a network profile because you no longer need it or because the password or connection scheme have changed and you need to recreate it.

To delete a network profile, follow these steps:

- From the Utilities menu, use the Left or  Right arrow to navigate to Wireless LAN information and press "Select".

- The "Network Setup" dialog box is opened.

- If Wireless is off, use the Left or Right arrow to change the status to On.

- Press the Down arrow.

- The list of available networks is displayed.

- Use the Left or Right arrow to navigate to the network you want to delete.

- Press and hold the "Cancel" key for more than 2 seconds. The unit announces, "Delete profile completed". If you are currently connected to the network, you are disconnected, and the profile is removed.

14. Appendices:

14.1. Appendix 1: The premium Accessory Pack.

- The Blaze EZ Premium Accessory pack includes the following items:

- OCR Stand

- OCR Remote

- Cradle

- Spare AC Adapter

- Spare Battery Pack

14.1.1 Assembling the OCR stand.

- The OCR stand is shipped in several pieces to make the shipping package a more manageable size. The package should include 2 identical small nuts, and 3 metal pieces of varying shapes and sizes. To assemble the OCR stand, follow these steps:

- Locate the long flat plate with the lip in the center, and place it on a desk or table top in front of you.

- Locate a second long plate with a small bolt near  each end. Place it standing upright flat against the inner wall of the lip on the first plate so that the ridge running down its center is facing inward.

- The small bolt in the bottom end of the second plate should slide into a small hole in the center of the lip of the first plate.

- Place 1 of the nuts over the bolt and screw it tight.

- Locate the small rectangular dish with the lip protruding from its bottom.

- Place the lip over the upper end of the second plate so that the plate is on the inside of the lip.

- The bolt near the top of the second place should slide into the small hole in the center of the lip of the dish.

- Place the second nut over the bolt protruding through the lip on the dish and screw it tight.

14.1.2 The OCR remote.

- The OCR remote allows you to control OCR without directly touching the Blaze EZ in order to maximize the stability of the camera when taking a picture.

- Hold the remote in your hand so that the end with the micro USB port is facing you. The remote contains 2 buttons. The top most button is the "OCR" button, and the bottom most button is the "Cancel" button. These buttons have the same functions as the equivalent buttons on the Blaze EZ.

- To use the OCR remote, connect 1 end of the included remote cable to the micro USB port on the bottom center of the Blaze EZ. Connect the other end to the Micro USB port on the bottom center of the OCR remote.

14.1.3 The Cradle

- The cradle accessory allows you to charge your Blaze EZ as well as a spare battery. It also allows you to connect external speakers more easily.

- Place the cradle on a desk or table top so that the ports are facing to the back. There is a tall rectangular protusion facing toward you into which you can insert the Blaze EZ.

- Inside this compartment are 2 connectors. On the left is an audio connector and on the right is a micro USB connector. To use the Blaze EZ in the cradle, slide it into the compartment with the keyboard facing toward you. Gently press down until you feel the connectors engage.

- Before using the cradle", connect the AC adapter to the micro USB port near the left end of the back of the cradle. Connect external speakers to the audio jack nearer the right end of the back of the cradle.

- When the Blaze EZ is connected to the cradle it charges via the USB port, and audio outputs via the external speakers connected to the cradle. If you are using a Bluetooth speaker, the audio output method is changed to the auxiliary external speaker connected to the cradle. When you disconnect the Blaze EZ from the cradle, audio output is returned to the Bluetooth headphones or speaker, provided it is still powered on and in range.

- To charge the spare battery, place it in the flat compartment behind the Blaze EZ compartment so that the small dot is on the top surface of the battery near the far left corner. Place the left end in first, and gently press the right end in place.

14.2. Appendix 2: Using Blaze EZ with Windows Media Player and Windows XP.

- This chapter describes how to install and use Windows Media Player 11 for use with Windows XP and Blaze EZ.

- To install Windows Media Player 11, follow the procedure below:

- Launch your web browser and connect to the link below.

- http://www.microsoft.com/ko-kr/download/windows-media-player-details.aspx

- "tab" to "download", find and click the link.

- When the download is complete, run the installation file.

- The "Genuine Windows Validation" dialog box is displayed. "Tab" to the "OK" button and press "Space".

- The "software License Agreement" dialog box is displayed. "Tab" to the "Agree" button and press "Space".

- When the installation is complete, press "Space" on the "Finish" button.  You may now use your Blaze EZ with Windows Media Player 11 after powering on and connecting it to the PC via a USB cable.

- Note: If you have installed Windows Media Player 11, yet the Blaze EZ is still not recognized, you will need to install the driver manually.

- Installation instructions are as follows:

- Open the ""control Panel".

- Navigate to "System" and press "Control-Tab" twice to move to the "Hardware" tab.

- "tab" to "Device Manager" and press "Enter". the.

- Click "OK". A list of devices currently installed on your PC is displayed.

- Press the Down arrow to navigate to 'Universal Serial Bus controllers' tree view.

- Press the Right arrow to expand the tree view and locate the "USB Mass Storage device".

- Open the menu.

- Press the up or down arrow to navigate to "Update Driver" and press "Enter" to run it.

- The "New Hardware Wizard" appears. Choose "this time only" and press "Enter" on the "Next button".

- "Install the software automatically" appears, press the down arrow key to" navigate to "Install from a list or specific location" and press "Enter".

- The Search window appears. Press the down arrow to navigate to "Do not search. I will choose the driver to install" and press "Enter".

- Under "Show compatible hardware", Check the box that appears under the tab key, and press the arrow keys to navigate to "Mtp Driver" and press "Enter".

- The "Installation Complete" message should be displayed shortly. Press "Enter" on the "Finish" button 'to complete the installation.